HEAT AND MASS FLOW PROCESSES

laboratory MAnual

UET

Department of Mechanical engineering (KSK CAMPUS)

University of engineering & science technology

Lahore

Engr. ADNAN QAMAR

(BSc. Mechanical Engineering UET, Lahore)

(MSc. Thermal Power Engineering UET, Lahore)

Submitted by:

Name =

Registration No =

Submitted to:

Mr. Adnan Qamar

# Preface

In most of the technology institutions, the laboratory course forms an integral form of the basic course in Heat and Mass Flow Processes at undergraduate level. The experiments to be performed in a laboratory should ideally be designed in such a way as to reinforce the understanding of the basic principles as well as help the students to visualize the various                            phenomenon encountered in different applications.

The objective of this manual is to familiarize the students with practical skills, measurement techniques and interpretation of results. It is intended to make this manual self-contained in all respects, so that it can be used as a laboratory manual.  In all the experiments, the relevant theory and general guidelines for the procedure to be followed have been given. Tabular sheets for entering the observations have also been provided in each experiment while graph sheets have been included wherever necessary.

The students are advised to refer to the relevant text before interpreting the results and writing a permanent discussion. The questions provided at the end of each experiment will reinforce the students understanding of the subject and also help them to prepare for viva-voce exams.

# General Instructions to Students

- The purpose of this laboratory is to reinforce and enhance your understanding of the fundamentals of Heat and mass transfer. The experiments here are designed to demonstrate the applications of the basic heat flow principles and to provide a more intuitive and physical understanding of the theory. The main objective is to introduce a variety of classical experimental and diagnostic techniques, and the principles behind these techniques. This laboratory exercise also provides practice in making engineering judgments, estimates and assessing the reliability of your measurements, skills which are very important in all engineering disciplines.

- Read the lab manual and any background material needed before you come to the lab.  You must be prepared for your experiments before coming to the lab.

- Actively participate in class and don’t hesitate to ask questions. Utilize the teaching assistants. You should be well prepared before coming to the laboratory, unannounced questions may be asked at any time during the lab.

- Carelessness in personal conduct or in handling equipment may result in serious injury to the individual or the equipment.  Do not run near moving machinery. Always be on the alert for strange sounds. Guard against entangling clothes in moving parts of machinery.

- Students must follow the proper dress code inside the laboratory.  To protect clothing from dirt, wear a lab apron. Long hair should be tied back.

- Calculator, graph sheets and drawing accessories are mandatory.

- In performing the experiments, proceed carefully to minimize any water spills, especially on the electric circuits and wire.

- Make your workplace clean before leaving the laboratory. Maintain silence, order and discipline inside the lab.

- Cell phones are not allowed inside the laboratory.

- Any injury no matter how small must be reported to the instructor immediately.

- Wish you a wonderful experience in this lab

# Table of Contents

Preface	3

General Instructions to Students	4

Table of Contents	5

List of Equipment	14

List of Experiments	15

List of Figures	16

List of Tables	17

List of Graphs	18

1.	LAB SESSION 1	19

1.1	Learning Objectives	20

1.2	Apparatus	20

1.3	Main Parts of Linear Heat Transfer Unit	20

1.4	Useful Data	20

1.5	Theory	20

1.5.1	Conduction Heat Transfer	21

1.5.2	Fourier’s law of heat conduction	21

1.5.3	The Plane walls	21

1.6	Experimental Procedure	21

1.7	Observations	22

1.8	Calculated Data	22

1.8.1	Objective:1	23

1.8.1.1	Specimen Calculations	23

1.8.1.2	Graph:	24

1.8.1.3	Statistical Analysis	24

1.8.1.4	Conclusion	24

1.8.2	Objective:2	25

1.8.2.1	Specimen Calculations	25

1.8.2.2	Statistical Analysis	26

1.8.2.3	Conclusion	27

1.9	Questions	27

1.10	Comments	27

2.	LAB SESSION 2	27

2.1	Learning Objective	28

2.2	Apparatus	28

2.3	Main Parts of Linear Heat Transfer Unit	28

2.4	Useful Data	28

2.5	Theory	29

2.5.1	Conduction Heat Transfer	29

2.5.2	Fourier’s law of heat conduction	29

2.5.3	The Composite wall	29

2.6	Experimental Procedure	30

2.7	Observations	31

2.7.1.1	Specimen Calculations	32

2.7.1.2	Graph	33

2.7.1.3	Statistical Analysis	34

2.7.1.4	Conclusion	34

2.8	Questions	34

2.9	Comments	34

3.	LAB SESSION NO 03	35

3.1	Learning Objective	35

3.2	Apparatus	35

3.3	Main Parts of Radial Heat transfer unit	35

3.4	Useful Data	35

3.4.1	Radial Heat Conduction	35

3.4.2	Heated disc:	35

3.5	Theory	35

3.5.1	Conduction Heat Transfer	35

3.5.2	Radial Systems	35

3.5.2.1	Cylinders	35

3.6	Procedure	36

3.7	Observations	37

3.8	Calculated Data	37

3.8.1	Objective	37

3.8.1.1	Specimen Calculations	38

3.8.1.2	Conclusion	38

3.8.1.3	Graph	39

3.9	Statistical Analysis	39

3.10	Questions	39

3.11	Comments	39

4.	LAB SESSION 4	40

4.1	Learning Objectives	40

4.2	Apparatus	40

4.3	Main Parts	40

4.4	Useful Data	40

4.5	Theory	40

4.5.1	Radiation	40

4.5.2	Radiation Heat Transfer	40

4.5.3	Inverse square law	41

4.5.4	Stefan–Boltzmann law	41

4.6	Procedure	41

4.7	Observations:	43

4.8	Calculated Data	43

4.8.1	Objective:01	43

4.8.1.1	Graph	44

4.8.1.2	Conclusion	44

4.8.2	Objective:02	44

4.8.2.1	Specimen Calculation	44

4.8.2.2	Conclusion	45

4.9	Statistical Analysis	45

4.10	Questions	45

4.11	Comments	45

5.	LAB SESSION 5	46

5.1	Apparatus	46

5.2	Main Parts	46

5.3	Useful Data	46

5.4	Theory	46

5.4.1	Radiation	46

5.4.2	Shape Factor	46

5.5	Procedure	47

5.6	Observations:	48

5.6.1	Calculated Data	49

5.6.1.1	Specimen Calculation	49

5.6.1.2	Conclusion	50

5.7	Statistical Analysis	50

5.8	Questions	50

5.9	Comments	50

6.	LAB SESSION 6	51

6.1	Learning Objective	51

6.2	Apparatus	51

6.3	Main Parts	51

6.4	Useful Data	51

6.5	Theory	52

6.6	Procedure	52

6.7	Calculated Data	54

6.7.1	Objective:01	54

6.7.1.1	Observations	54

6.7.1.2	Specimen Calculations	54

6.7.1.3	Graph	55

6.7.1.4	Conclusion	55

6.8	Statistical Analysis	56

6.9	Questions	56

6.10	Comments	56

7.	LAB SESSION 7	57

7.1	Learning Objective	57

7.2	Apparatus	57

7.3	Main Parts	57

7.4	Useful Data	57

7.5	Theory	58

7.5.1	Forced convection	58

7.6	Procedure	58

7.6.1	Objective	60

7.6.1.1	Observations	60

7.6.1.2	Calculated Data	60

7.6.1.3	Specimen Calculations	60

7.6.1.4	Graph:	62

7.6.1.5	Conclusion	62

7.7	Statistical Analysis	62

7.8	Questions	62

7.9	Comments	62

8.	LAB SESSION 8	66

8.1	Learning Objective	66

8.2	Apparatus	66

8.3	Main Parts	66

8.4	Useful Data	66

8.5	Theory:	66

8.5.1	Fin (extended surface)	66

8.6	Procedure:	67

8.7	Observations	68

8.8	Calculated Data	69

8.8.1	Specimen Calculation	69

8.8.2	Graph	70

8.8.3	Conclusion	70

8.9	Questions	70

8.10	Comments	70

9.	LAB SESSION 9	71

To Reduce radiation errors in measurement of temperature by using shield between sensor and source of radiation	71

9.1	Learning Objective	71

9.2	Apparatus	71

9.3	Main Parts	71

9.4	Theory	71

9.5	Procedure:	72

9.6	Observations	73

9.6.1	Objective	73

9.6.1.1	Graph	74

9.6.1.2	Conclusion	74

9.7	Comments	74

10.	LAB SESSION 10	75

10.1	Learning Objective	75

10.2	Apparatus	75

10.3	Main Parts	75

10.4	Theory	75

10.4.1	Transient Heat-Transfer	75

10.4.2	Lumped-Heat-Capacity System	75

10.4.3	Biot number (Bi)	75

10.4.4	Fourier number (Fo)	76

10.4.5	Heisler charts	77

10.5	Procedure	77

10.6	Observations	79

10.7	Calculated Data	80

10.7.1	Sample Calculations	80

10.7.2	Graph	81

10.7.3	Conclusion	82

10.8	Statistical Analysis	82

10.9	Questions	82

10.10	Comments	82

11.	LAB SESSION 11	84

11.1	Learning Objective	84

11.2	Apparatus	84

11.3	Main Parts	84

11.4	Useful Data	84

11.5	Theory	85

11.5.1	Introduction of The Equipment	85

11.5.2	Description of The Equipment	86

11.6	Objective: 1	86

11.6.1	Procedure	86

11.6.2	Observations	87

11.6.3	Graph	87

11.6.4	Conclusion	87

11.7	Objective: 2	88

11.7.1	Procedure	88

11.7.2	Observations:	88

11.7.3	Graph	88

11.7.4	Conclusion	88

11.8	Specimen Calculation	88

11.9	Statistical Analysis	89

11.10	Questions	89

11.11	Comments	89

12.	LAB SESSION 12	90

12.1	Learning Objective	90

12.2	Apparatus	90

12.3	Main Parts	90

12.4	Useful Data	91

12.4.1	Interior Tube	91

12.4.2	Exterior Tube	91

12.4.3	Base Unit	91

12.4.4	Heat Exchanger	91

12.4.5	Physical Properties of The Hot And Cold Water	91

12.4.6	Mass Flow Rates	91

12.5	Theory	92

12.5.1	Heat transference in heat exchangers	92

12.5.2	Overall Heat-Transfer Coefficient	93

12.5.3	The Log Mean Temperature Difference (LMTD)	93

12.5.4	Effectiveness-Ntu Method	93

12.5.5	Capacity coefficient	93

12.5.6	Reynolds number	94

12.6	Objective 01	94

12.6.1	Procedure:	94

12.6.2	Observations	95

12.6.3	Calculated Data	95

12.6.4	Specimen Calculations	96

12.6.4.1	Heat transferred by the hot water	96

12.6.4.2	Heat absorbed by the cold water	96

12.6.4.3	Heat Losses	96

12.6.4.4	Logarithmic temperatures mean difference between hot water and cold water	96

12.6.4.5	Global heat transference coefficient= (U)	96

12.7	Objective:02	96

12.7.1	Procedure	96

12.7.2	Observations	97

12.7.3	Calculations	98

12.7.4	Specimen Calculations	98

12.7.4.1	Heat transferred by the hot water	98

12.7.4.2	Logarithmic temperatures mean difference between hot water and cold water	98

12.7.4.3	Number of transmission units	99

12.7.4.4	Capacity coefficient	99

12.8	Graph	99

12.9	Statistical Analysis	99

12.10	Conclusion	99

12.11	Questions	100

12.12	Comments	100

13.	LAB SESSION 13	101

13.1	Learning Objective	101

13.2	Apparatus	101

13.3	Main Parts	101

13.4	Useful Data	102

13.4.1	Interior Tube	102

13.4.2	Exterior Tube	102

13.4.3	Base Unit	102

13.4.4	Heat Exchanger	102

13.4.5	Physical Properties Of The Hot And Cold Water	102

13.4.6	Mass Flow Rates	103

13.5	Theory	104

13.5.1	Heat transference in heat exchangers	104

13.5.2	Shell and tube heat exchanger	104

13.5.3	Overall Heat-Transfer Coefficient	104

13.5.4	The Log Mean Temperature Difference (LMTD)	105

13.5.5	Effectiveness-Ntu Method	105

13.5.6	Capacity coefficient	105

13.5.7	Reynolds number	105

13.6	Objective 1	106

13.6.1	Procedure:	106

13.6.2	Observations	106

13.6.3	Calculated Data	107

13.6.4	Specimen Calculations	107

13.6.4.1	Heat transferred by hot water (qh)	107

13.6.4.2	Heat absorbed by the cold water (qc)	107

13.6.4.3	Heat losses (ql)	107

13.6.4.4	Logarithmic average temperatures difference between hot and cold water (∆Tlm)	107

13.6.4.5	Global heat transfer coefficient (U)	107

13.7	Objective 2	108

13.7.1	Procedure	108

13.7.2	Observations	108

13.7.3	Calculated Data	109

13.7.4	Specimen Calculations	109

13.7.4.1	Experimental effectiveness (ϵ)	109

13.7.4.2	Logarithmic average temperatures difference between hot and cold water (∆Tlm)	109

13.7.4.3	Global heat transfer coefficient (U)	109

13.7.4.4	Number of transmission units	109

13.7.4.5	Capacity coefficient	110

13.7.4.6	Temperatures at the exchanger outlet	110

13.8	Statistical Analysis	110

13.9	Conclusion	110

13.10	Questions	110

13.11	Comments	110

14.	LAB SESSION 14	111

14.1	Learning Objective	111

14.2	Apparatus	111

14.3	Main Parts	111

14.4	Useful Data	111

14.4.1	BASE UNIT	111

14.4.2	Heat Exchanger	111

14.4.3	Physical Properties of The Hot and Cold Water	112

14.4.4	Mass Flow Rates	112

14.5	Theory	114

14.5.1	Heat transference in heat exchangers	114

14.5.2	Plate heat exchanger	114

14.5.3	Overall Heat-Transfer Coefficient	114

14.5.4	The Log Mean Temperature Difference (LMTD)	114

14.6	Objective	115

14.6.1	Procedure	115

14.6.2	Observations	115

14.6.3	Calculated Data	116

14.6.4	Specimen Calculations	116

14.6.4.1	Heat transferred by hot water (qh)	116

14.6.4.2	Heat absorbed by the cold water (qc)	116

14.6.4.3	Heat losses (ql)	116

14.6.4.4	Logarithmic average temperatures difference between hot and cold water (∆Tlm)	116

14.6.4.5	Global heat transfer coefficient (U)	116

14.7	Graph	117

14.8	Statistical Analysis	117

14.9	Conclusion	117

14.10	Questions	117

14.11	Comments	117

# List of Equipment

- Linear Heat Transfer Unit (H111A)

- Radial Heat Transfer Unit (H111B)

- Radiation Heat Transfer Unit (H111C)

- Combined Convection & Radiation Heat Transfer Unit (H111D)

- Extended Surface Heat Transfer Unit (H111E)

- Radiation Errors in Temperature Measurements (H111F)

- Heat Transfer Service Unit with Unsteady State Heat Transfer Unit (H111G)

- Free and Forced Convection Heat Transfer Unit (TCLFC)

- Turbulent Flow Heat Exchanger (TIFT)

- Shell and Tube Heat Exchanger (TICT)

- Plate Heat Exchanger (TIPL)

# List of Experiments

| Experiment No. | Description |
| --- | --- |
| Experiment No. 1 | To apply Fourier Rate Equation for steady flow of heat through plane solid materials. |
| Experiment No. 2 | To apply Fourier Rate Equation for steady flow of heat through composite plate |
| Experiment No. 3 | To apply the Fourier Rate Equation for steady flow of heat through cylindrical solid materials |
| Experiment No. 4 | To verify laws of radiation by using shape factor |
| Experiment No. 5 | To verify the role of shape factor involved in radiation heat transfer |
| Experiment No. 6 | To calculate the total heat transfer of combined convection and radiation heat transfer mechanism |
| Experiment No. 7 | To calculate the total heat transfer of combined “forced convection” and “radiation” heat transfer mechanism |
| Experiment No. 8 | To experimentally verify the heat transfer from an extended surface from combined modes of free conduction, free convection and radiation heat transfer by comparing it with the theoretical analysis |
| Experiment No. 9 | To Reduce radiation errors in measurement of temperature by using shield between sensor and source of radiation |
| Experiment No. 10 | To determine the Convective heat transfer coefficient of a solid cylinder using analytical transient temperature/heat flow charts |
| Experiment No. 11 | To measure the effect of different exchanger geometry on convection heat transfer mechanism |
| Experiment No. 12 | To Calculate logarithmic mean temperature difference (LMTD), global heat transfer coefficient and effectiveness of turbulent flow heat exchanger |
| Experiment No. 13 | To Calculate logarithmic mean temperature difference (LMTD), global heat transfer coefficient and effectiveness of shell and tube heat exchanger |
| Experiment No. 14 | To Calculate logarithmic mean temperature difference (LMTD) and global heat transfer coefficient of plate heat exchanger |

# List of Figures

Figure 1.1: Temperature Profile in Fourier heat conduction	22

Figure.1.2:  Schematic diagram of experiment	24

Figure.1.3:  Schematic diagram of experiment	25

Figure 1.4:  Schematic diagram of experiment	26

Figure 2.1: Temperature Profile in Fourier heat conduction	30

Figure 2.2: Heat through composite wall	31

Figure 2.3:  Schematic diagram of experiment	33

Figure 2.4: Schematic diagram of experiment	34

Figure 3.1: Heat transfer through cylinder	37

Figure 3.2: Temperature distribution in cylinder	37

Figure 4.1:  Schematic diagram of experiment	43

Figure.5.1:  Schematic diagram for shape factor	48

Figure 5.2:  Schematic diagram of experiment	49

Figure 6.1:  Schematic Diagram of Experiment	55

Figure 7.1:  Schematic Diagram of Experiment	61

Figure 8.1:  Schematic Diagram of Experiment	70

Figure 8.1:  Schematic Diagram of Experiment	87

Figure 12.1: Schematic Diagram of Experiment	94

Figure 13.1: Schematic Diagram of Experiment	105

Figure 13.2: Parallel and counter parallel flow	106

Figure 13.3: Shell and tube heat exchanger	106

Figure 14.1: Schematic Diagram of Experiment	115

Figure 14.2: Parallel and counter parallel flow	116

# List of Tables

Table 1.1: Temperatures observation at different points of specimen	24

Table 1.2: Calculation of thermal conductivity	24

Table 1.3: Distribution of temperature through uniform plane wall	26

Table 2.1: Temperatures observation at different points of specimen	32

Table 2.2: Calculation of overall heat transfer coefficient	33

Table 3.1: Distribution of temperature for different voltages	38

Table 3.2: Thermal conductivity of given sample	39

Table 4.1: Measurement of temperatures from radiometer	44

Table 4.2: Calculation of intensity of radian	45

Table 4.3: Calculation of shape factor	46

Table 5.1: Measurement of temperatures from radiometer	50

Table 5.2: Radiation incident on the detector	51

Table 6.1: Table of Physical properties of Air at Atmospheric Pressure	53

Table 6.2: Measurement of temperature for free convection	56

Table 6.3: Heat transfer due to free convection and radiation	56

Table 7.1: Table of Physical properties of Air at Atmospheric Pressure	59

Table 7.2: Measurement of temperature due to forced convection	62

Table 7.3: Heat transfer due to forced convection	62

Table 8.1: Temperatures measured at different distances from T1	70

Table 8.2: Heat transfer from extended surface	71

Table 9.1: Temperatures at different points from radiant source	75

Table 10.1: Specimen: 20mm diameter Brass Cylinder.	81

Table 10.2: Specimen: 20mm Stainless Steel Cylinder	81

Table 10.3:  Properties of Specimen: 20mm diameter Brass Cylinder	82

Table 10.4:  Properties of Specimen: 20mm diameter stainless steel Cylinder	82

Table 11.1: Temperatures in free convection	89

Table 11.2: Temperatures in forced convection	89

Table 11.3: Temperatures for different geometry of exchangers	90

Table 12.1: Temperature at different points of turbulent flow heat exchanger	97

Table 12.2: Heat transfer calculation in turbulent flow heat exchanger	97

Table 12.3: Temperature for parallel and cross flow	99

Table 12.4: Effectiveness of turbulent flow exchanger	100

Table 13.1: Valve positions of shell and tube heat exchanger	105

Table 13.2: Temperatures at shell and tube heat exchanger	108

Table13.3: Heat transfer coefficient of shell and tube heat exchanger	109

Table 13.4: Temperatures for shell and tube heat exchanger	110

Table 13.5: Effectiveness of shell and tube heat exchanger	111

Table 14.1: Position of valves in plate heat exchanger	115

Table 14.2: Temperatures of Plate Heat Exchangers	117

Table14.3: Heat transfer in Plate heat exchanger	118

# List of Graphs

Graph 7.1: Kinematic velocity of air at standard Pressure	65

Graph 7.2: Thermal conductivity of air at standard pressure	66

Graph 7.3:  Prandtl number of air at standard pressure	67

Graph 10.1: Heisler charts	85

# LAB SESSION 1

To apply Fourier Rate Equation for steady flow of heat through plane solid materials.

## Learning Objectives

- To identify a given sample by determining its thermal conductivity.

- To measure temperature distribution through a uniform plane wall and analyse the effect of a change in heat flow.

## Apparatus

Linear Heat Transfer unit H111A (Serial no H111A/04417)

## Main Parts of Linear Heat Transfer Unit

- Hydraulic Bench

- Specimen of different materials (Brass, Aluminium, Stainless Steel)

- Main digital Control panel (H111)

- Temperature Sensors

## Useful Data

Heated Section:

Material: Brass 25 mm diameters, Thermocouples T1, T2, T3 at 15mm spacing. Thermal Conductivity: Approximately 121 W/mK

Cooled Section:

Material: Brass 25 mm diameters, Thermocouples T6, T7, T8 at 15mm spacing. Thermal Conductivity: Approximately 121 W/mK

Brass Intermediate Specimen:

Material: Brass 25 mm diameters * 30mm long. Thermocouples T4, T5 at 15mm spacing centrally spaced along the length.  Thermal Conductivity: Approximately 121 W/mK

Hot and Cold Face Temperature:

Due to the need to keep the spacing of the thermocouples constant at 15mm with, or without the intermediate specimens in position the thermocouples are displaced 7.5 mm back from the ends faces of the heated and cooled specimens and similarly located for the Brass Intermediate Specimen.

T hot face = T3-(T2-T3)/2            T cold face = T6 + (T6-T7)/2

So that the equations are of the above form as the distance between T3 and the hot face and T6 and the cold face are equal to half the distance between the adjacent pairs of thermocouples.

## Theory

### Conduction Heat Transfer

When a temperature gradient exists in a body, there is an energy transfer from the high-temperature region to the low-temperature region. The energy is transferred by conduction and that the heat-transfer rate per unit area is proportional to the normal temperature gradient

### Fourier’s law of heat conduction

Figure 1.1: Temperature Profile in Fourier heat conduction

qx=-KA∂T∂x           Equation I.1: Fourier law heat conduction equation

Where qx is the heat-transfer rate and ∂T/∂x is the temperature gradient in the direction of the heat flow. The positive constant k is called the thermal conductivity of the material, and the minus sign is inserted so that the second principle of thermodynamics will be satisfied; i.e., heat must flow downhill on the temperature scale as shown in figure 1.1. Equation 1.1 is called Fourier’s law of heat conduction after the French mathematical physicist Joseph Fourier, who made very significant contributions to the analytical treatment of conduction heat transfer. (Equation 1.1) is the defining equation for the thermal conductivity and that k has the units of watts per meter per Cels1ius degree in a typical system of units in which the heat flow is expressed in watts

### The Plane walls

First consider the plane wall where a direct application of Fourier’s law. Integration yields

q=-KA∆x (T2-T1)                             Equation 1.2: Integration of Fourier law

When the thermal conductivity is considered constant. The wall thickness is x, and T1 and T2 are the wall-face temperatures.

## Experimental Procedure

- Ensure that the main switch is in the off position (the digital displays should not be illuminated). Ensure that the residual current circuit breaker on the rear panel is in the ON position.

- Turn the voltage controller anti-clockwise to set the AC voltage to minimum.

- Ensure the cold-water supply and electrical supply is turned on at the source. Open the water tap until the flow through the drain hose is approximately 1.5 litters/minute.

- Release the toggle clamp tensioning screw and clamps. Ensure that the faces of the exposed ends of the heated and cooled sections are clean.

- Turn on the main switch and the digital displays should illuminate. Set the temperature selector switch to T1 to indicate the temperature of the heated end of the bar.

- Following the above procedure ensure cooling water is flowing and then set the heater voltage V to approximately 150 volts. This will provide a reasonable temperature gradient along the length of the bar. If however the local cooling water supply is at a high temperature (25-35℃ or more) then it may be necessary to increase the voltage supplied to the heater. This will increase the temperature difference between the hot and cold ends of the bar.

- Monitor temperatures T1, T2, T3, T4, T5, T6, T7, T8 until stable. When the temperatures are stabilized record T1, T2, T3, T4, T5, T6, T7, T8, V, I

- Increase the heater voltage by approximately 50 volts and repeat the above procedure again recording the parameters T1, T2, T3, T4, T5, T6, T7, T8, V, I when temperature have stabilized.

- When the experimental procedure is completed, it is good practice to turn off the power to the heater by reducing the voltage to zero and allow the system a short time to cool before turning off the cooling water supply.

## Observations

| Sample No. | T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | V | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | °C | °C | °C | °C | °C | °C | °C | °C | Volts | Amps |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
| Distance from T1 | 0.000 | 0.015 | 0.030 | 0.045 | 0.060 | 0.075 | 0.090 | 0.105 |  |  |

##### Table 1.1: Temperatures observation at different points of specimen

## Calculated Data

### Objective:1

To identify a given sample by determining its thermal conductivity.

###### Figure.1.2:  Schematic diagram of experiment

Specimen cross sectional Area A=0.00049m2

Specimen Length =0.030m

| Sample No. | Q | Thotface | Tcoldface | ∆Tint | kint | Material Name |
| --- | --- | --- | --- | --- | --- | --- |
|  | Watts | ℃ | ℃ | K | W/mK |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

##### Table 1.2: Calculation of thermal conductivity

#### Specimen Calculations

Intermediate Specimen and hot and cold section cross sectional Area;

A=πD24

Heat transfer rate from the heater;

Q=V*I

Note that the thermocouples T3 and T6 do not record the hot face and cold face temperatures, as are both displaced by 0.0075m from T3 and T6 as shown.

###### Figure.1.3:  Schematic diagram of experiment

If it is assumed that the temperature distribution is linear, then the actual temperature at the hot face and cold face may be determined from the following equations.

T hot face = T3-(T2-T3)/2

And

T cold face = T6 + (T6-T7)/2

∆Tint=Thotface_Tcoldface

From the above parameters, the thermal conductivity of the aluminum intermediate section may be calculated.

kint=Q∆xintAint(Thotface-Tcoldface)=Q∆xintAint∆Tint

#### Graph:

Plot a graph b/w Temperature and Distance from T1 thermocouple. The thermal conductivity of the intermediate sample may also be calculated from the data it is plotted on a graph. From the graph the slope of the line is.

∆Tint∆xint=X

kint=QA*∆Tint∆xint

#### Statistical Analysis

For Thermal conductivity

% Error= Kth-KexpKth

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

#### Conclusion

### Objective:2

To measure the temperature distribution through a uniform plane wall and demonstrate the effect of a change in heat flow.

###### Figure 1.4:  Schematic diagram of experiment

| Sample No. | Q | ∆T 1-3 | ∆T 6-8 | ∆x 1-3 | ∆x 6-8 | ∆T1-3∆x1-3 | ∆T6-8∆x6-8 | Q/(∆T1-3∆x1-3) | Q/(∆T6-8∆x6-8) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W | ℃ | ℃ | m | M | ℃/m | ℃/m | W/mK | W/mK |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |

##### Table 1.3: Distribution of temperature through uniform plane wall

#### Specimen Calculations

Heat transfer rate from the heater;

Q=V*I

Temperature difference in the heated section between T1 and T3;

∆Thot=∆T1-3=T1-T3

Similarly the temperature difference in the cooled section between T6 and T8;

∆Tcold=∆T6-8=T6-T8

The distance between the temperatures measuring points, T1 and T3and T6 and T8, are similar;

∆x1-3=

∆x6-8=

Hence the temperature gradient along the heated and cooled sections may be calculated from

HeatedSection=∆T1-3∆x1-3=

CooledSection=∆T6-8∆x6-8=

If the constant rate of heat transfer is divided by the temperature gradients, the value obtained will be similar if the equation is valid.

Q=C∆T∆x

Q∆T∆x=C

Hence, substituting the values obtained gives for the heated section and cooled sections respectively for following values.

Q∆T1-3∆x1-3=

Q∆T6-8∆x6-8=

As may be seen from the above example and the tabulated data the function does result in a constant value within the limits of the experimental data.

Q∆T∆x=C

#### Statistical Analysis

For Constant “C”

% Error= Kth-KexpKth

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

#### Conclusion

## Questions

- What is conduction heat transfer?

- What is Fourier law of heat conduction

- What is meant by thermal resistance

## Comments

# LAB SESSION 2

To apply Fourier Rate Equation for steady flow of heat through composite plate

## Learning Objective

- To measure the temperature distribution through a composite plane wall and determine the overall Heat Transfer Coefficient for the flow of heat through a combination of different materials

## Apparatus

Linear Heat Transfer unit H111A (Serial no H111A/04417)

## Main Parts of Linear Heat Transfer Unit

- Hydraulic Bench

- Specimen of different materials (Brass, Aluminium, Stainless Steel)

- Main digital Control panel (H111)

- Temperature Sensors

## Useful Data

Heated Section:

Material: Brass 25 mm diameters, Thermocouples T1, T2, T3 at 15mm spacing. Thermal Conductivity: Approximately 121 W/mK

Cooled Section:

Material: Brass 25 mm diameters, Thermocouples T6, T7, T8 at 15mm spacing. Thermal Conductivity: Approximately 121 W/mK

Brass Intermediate Specimen:

Material: Brass 25 mm diameters * 30mm long. Thermocouples T4, T5 at 15mm spacing centrally spaced along the length.  Thermal Conductivity: Approximately 121 W/mK

Stainless Steel Intermediate Specimen:

Material: Stainless Steel, 25 mm diameters * 30mm long. No Thermocouples fitted.

Thermal Conductivity: Approximately 25 W/mK

Aluminum Intermediate Specimen:

Material: Aluminum Alloy, 25 mm diameters * 30mm long. No Thermocouples fitted.

Thermal Conductivity: Approximately 180 W/mK

Hot and Cold Face Temperature:

Due to the need to keep the spacing of the thermocouples constant at 15mm with, or without the intermediate specimens in position the thermocouples are displaced 7.5 mm back from the ends faces of the heated and cooled specimens and similarly located for the Brass Intermediate Specimen.

T hot face = T3-(T2-T3)/2            T cold face = T6 + (T6-T7)/2

So that the equations are of the above form as the distance between T3 and the hot face and T6 and the cold face are equal to half the distance between the adjacent pairs of thermocouples.

## Theory

### Conduction Heat Transfer

When a temperature gradient exists in a body, there is an energy transfer from the high-temperature region to the low-temperature region. The energy is transferred by conduction and that the heat-transfer rate per unit area is proportional to the normal temperature gradient

### Fourier’s law of heat conduction

Figure 2.1: Temperature Profile in Fourier heat conduction

qx=-KA∂T∂x           Equation 2.1: Fourier law heat conduction equation

Where qx is the heat-transfer rate and ∂T/∂x is the temperature gradient in the direction of the heat flow. The positive constant k is called the thermal conductivity of the material, and the minus sign is inserted so that the second principle of thermodynamics will be satisfied; i.e., heat must flow downhill on the temperature scale as shown in figure 1.1. Equation 1.1 is called Fourier’s law of heat conduction after the French mathematical physicist Joseph Fourier, who made very significant contributions to the analytical treatment of conduction heat transfer. (Equation 1.1) is the defining equation for the thermal conductivity and that k has the units of watts per meter per Cels1ius degree in a typical system of units in which the heat flow is expressed in watts

### The Composite wall

If more than one material is present, as in the multilayer wall shown in Figure 2, the analysis would proceed as follows: The temperature gradients in the three materials are shown, and the heat flow may be written

q=−kAAT2-T1∆xA= −kBAT3-T2∆xB= −kCAT4-T3∆xC               Equation 2.2: Heat through composite wall

Note that the heat flow must be the same through all sections

###### Figure 2.2: Heat through composite wall

Solving these three equations simultaneously, the heat flow is written

q=T1-T4∆xAkAA+∆xBkBAA+∆xCkCA                      Equation 2.3: Heat flow equation through composite wall

With composite systems, it is often convenient to work with an overall heat transfer coefficient U, which is defined by an expression analogous to Newton’s law of cooling. Accordingly

Q=UA∆T                             Equation 2.4:  Overall heat transfer coefficient

Where ∆T is the overall temperature difference.  Readers can read Further detail from book “heat transfer by J.P Holman” in “chapter 2”

## Experimental Procedure

- Ensure that the main switch is in the off position (the digital displays should not be illuminated). Turn the voltage controller anti-clockwise to set the AC voltage to minimum.

- Ensure the cold-water supply and electrical supply is turned on at the source. Open the water tap until the flow through the drain hose is approximately 1.5 litters/minute.

- Release the toggle clamp tensioning screw and clamps. Ensure that the faces of the exposed ends of the heated and cooled sections are clean. Similarly, check the faces of the Intermediate specimen to be placed between the faces of the heated and cooled section.

- Turn on the main switch and the digital displays should illuminate. Set the temperature selector switch to T1 to indicate the temperature of the heated end of the bar.

- Following the above procedure ensure cooling water is flowing and then set the heater voltage V to approximately 150 volts. This will provide a reasonable temperature gradient along the length of the bar. If, however the local cooling water supply is at a high temperature (25-35℃ or more) then it may be necessary to increase the voltage supplied to the heater. This will increase the temperature difference between the hot and cold ends of the bar.

- Monitor temperatures T1, T2, T3, T4, T5, T6, T7, T8 until stable.

- Increase the heater voltage by approximately 50 volts and repeat the above procedure again recording the parameters T1, T2, T3, T4, T5, T6, T7, T8, V, I when temperature have stabilized.

## Observations

| Sample No. | T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | V | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | °C | °C | °C | °C | °C | °C | °C | °C | Volts | Amps |
| 1 |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |
| Distance from T1 | 0.000 | 0.015 | 0.030 | 0.045 | 0.060 | 0.075 | 0.090 | 0.105 |  |  |

##### Table 2.1: Temperatures observation at different points of specimen

To measure the temperature distribution through a composite plane wall and determine the overall Heat Transfer Coefficient for the flow of heat through a combination of different materials in use.

###### Figure 2.3:  Schematic diagram of experiment

Specimen cross sectional Area A = 0.00049m2

Conductivity of Brass heated and cooled section =121W/mK

Conductivity of Stainless steel intermediate section =25W/mK

| Sample No. | Q | ∆T1-8 | ∆xhot | ∆xint | ∆xcold | khot | kint | kcold |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -- | W | K | m | M | m | WmK | WmK | WmK |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |

| Sample No. | U=1(xhotkhot+xintkint+xcoldkcold) | QA(T1-T8)=U |
| --- | --- | --- |
| -- | Wm2K | Wm2K |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

##### Table 2.2: Calculation of overall heat transfer coefficient

#### Specimen Calculations

Brass Intermediate Specimen and hot and cold section cross sectional Area;

A=πD24

The temperature difference across the bar from T1 to T8;

T1-T8=X

Note that ∆xhot and ∆xcoldare the distances between the thermocouple T1 and the hot face and the cold face and the thermocouple T8 respectively. Similarly, ∆xint is the distance between the hot face and cold face of the intermediate stainless steel section.

###### Figure 2.4: Schematic diagram of experiment

The distances between surfaces are therefore as follows.

∆xhot = 0.0375m

∆xint = 0.030m

∆xcold= 0.0375m

Heat transfer rate from the heater;

Q=VI

Hence

U=QA(T1-T8)

Similarly, 				
U=1(xhotkhot+xintkint+xcoldkcold)

Note that the U value resulting from the test data differs from that resulting from assumed thermal conductivity and material thickness. This is most likely due to un-accounted for heat losses and thermal resistances between the hot face interface and cold face inter face with the stainless steel intermediate section.

#### Graph

Plot a graph b/w Temperature and Distance from T1 thermocouple. The temperature data may be plotted against position along the bar and straight lines drawn through the temperature points for the heated and cooled sections. Then a straight line may be drawn through the hot face and cold face temperature to extrapolate the temperature distribution in the stainless steel intermediate section.

#### Statistical Analysis

For Constant “U”

% Error= Kth-KexpKth

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

#### Conclusion

## Questions

- Define Overall heat transfer coefficient

- Define thermal conductivity and explain its significance in heat transfer.

## Comments

# LAB SESSION NO 03

To apply the Fourier Rate Equation for steady flow of heat through cylindrical solid materials

## Learning Objective

- To identify a given sample (disc material) by determining its thermal conductivity.

## Apparatus

Radial Heat Transfer unit H111B

## Main Parts of Radial Heat transfer unit

- Hydraulic Bench

- Specimen of different materials

- Main digital Control panel (H111)

- Temperature Sensors

## Useful Data

### Radial Heat Conduction

### Heated disc:

Material: Outside Diameter: 0.110m

Diameter of Heated Sample Core: 0.014m

Thickness of Disc (x): 0.0032m

Radial Position of Thermocouples:

T1=0.007m

T2=0.010m

T3=0.020m

T4=0.030m

## Theory

### Conduction Heat Transfer

When a temperature gradient exists in a body, there is an energy transfer from the high-temperature region to the low-temperature region. The energy is transferred by conduction and that the heat-transfer rate per unit area is proportional to the normal temperature gradient.

### Radial Systems

#### Cylinders

Consider a long cylinder of inside radius ri, outside radius ro, and length L, such as the one shown in Figure 2.1.We expose this cylinder to a temperature differential Ti −To. For a cylinder with length very large compared to diameter, it may be assumed that the heat flows only in a radial direction, so that the only space coordinate needed to specify the system is r. Again, Fourier’s law is used by inserting the proper area relation. The area for heat flow in the cylindrical system is

Ar =2πrL

So that Fourier’s law is written

qr= −kAr dTdr

qr= −2πkrL dTdr                Equation 3.1: Fourier Law equation for cylinders

###### Figure 3.1: Heat transfer through cylinder

###### Figure 3.2: Temperature distribution in cylinder

With the boundary conditions

T=Ti at r=ri

T=T0 at r=r0

The solution to Equation 3.1 is

q=2πkL(Ti-T°)ln⁡(r°ri)           Equation 3.2: Fourier Law equation for cylinders

## Procedure

- Ensure that the main switch is in the off position (the digital displays should not be illuminated).

- Ensure the cold water supply and electrical supply is turned on at the source. Open the water tap until the flow through the drain hose is approximately 1.5 liters/minute.

- Turn on the main switch and the digital displays should illuminate. Set the temperature selector switch to T1 to indicate the temperature of the heated centre of the disc.

- Following the above procedure ensure cooling water is flowing and then set the heater voltage V to approximately 50 volts.

- Monitor temperatures T1, T2, T3, T4, T5, and T6 until stable. When the temperatures are stabilized record: T1, T2, T3, T4, T5, T6, V, I

- Reset heater voltage to 100 volts and repeat the above procedure again and Monitor temperatures T1, T2, T3, T4, T5, and T6 until stable. When the temperatures are stabilized record: T1, T2, T3, T4, T5, T6 V, I. Repeat procedure with the difference of 50 V voltage

## Observations

| Sample No. | T1 | T2 | T3 | T4 | T5 | T6 | V | I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ⁰C | ⁰C | ⁰C | ⁰C | ⁰C | ⁰C | Volts | Amps |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| Radius | 0.007 | 0.010 | 0.020 | 0.030 | 0.040 | 0.050 | --- | --- |

##### Table 3.1: Distribution of temperature for different voltages

## Calculated Data

### Objective

To identify a given sample (disc material) by determining its thermal conductivity.

| Sample No. | Q | Q×ln(R6R1)2πx(T1-T6) | Q×ln(R3R1)2πx(T1-T3) | Q×lnR6R42πxT4-T6 |
| --- | --- | --- | --- | --- |
|  | Watts | W/mK | W/mK | W/mK |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |

##### Table 3.2: Thermal conductivity of given sample

#### Specimen Calculations

X=0.0032 (Thickness or length of disc)

Heat transfer rate from the heater;

Q=V*I

From Fourier’s equation (ignoring the negative sign)

k=QlnR°Ri2πx(T°-Ti)

Examining the points T1 and T6 and substituting values gives

k=Q×ln(R6R1)2πx(T1-T6)

Similarly, the other pair of points on Radius;

k=QlnR3R12πx(T3-T1)

k=QlnR6R42πx(T6-T4)

#### Conclusion

#### Graph

Plot a graph b/w Radius from heated disc centre (x-axis) and Temperature (y-axis).

## Statistical Analysis

For Thermal conductivity

% Error= Kth-KexpKth

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Questions

- How temperature distribution through the wall of a thick cylinder

- What is steady-state heat transfer?

- What is effect of changing the thickness of cylinder on thermal conductivity?

- Why by increasing input heat Q, overall slope of the temperature gradients increases.

## Comments

# LAB SESSION 4

To verify laws of radiation by using shape factor

## Learning Objectives

To verify the inverse square law for thermal radiation

To verify the Stefan Boltzmann Law

## Apparatus

Radiation Heat Transfer unit H111C (Serial no H111C/01310)

## Main Parts

- Heat Source

- Radiation detector

- Temperature Sensors

- Radiometer output unit

- Main digital Control panel (H111)

## Useful Data

Laws of radiant heat transfer and radiant heat exchange (H111C)

Stefen-Boltzman Constant σ =5.67x10-8 wm2k4

## Theory

### Radiation

Radiation is the emission or transmission of energy in the form of waves or particles through space or through a material medium.

### Radiation Heat Transfer

Thermal radiation is electromagnetic radiation generated by the thermal motion of charged particles in matter. All matter with a temperature greater than absolute zero emits thermal radiation. When the temperature of the body is greater than absolute zero, interatomic collisions cause the kinetic energy of the atoms or molecules to change. All bodies radiate energy in the form of photons moving in a random direction, with random phase and frequency. When radiated photons reach another surface, they may be absorbed, reflected or transmitted. The behavior of a surface with radiation incident upon it can be described by the following quantities:

= absorptance =fraction of incident radiation absorbed

Ρ= reflectance = fraction of incident radiation reflected

- = transmittance = fraction of incident radiation transmitted.

### Inverse square law

Intensity is inversely proportional to the square of the distance from the source of that physical quantity. Mathematically formulated:

Intensity∝1distance2

### Stefan–Boltzmann law

To show that the intensity of radiation varies as the fourth power of the source temperature. The Stefan-Boltzmann law states that for a black body

qb= σ (Ts4 -Ta4 )

Equation 4.1: Stefan-Boltzmann law

Where

qb          = The energy emitted per unit area of a black body radiator  Wm2

Ts           = The absolute temperature of the black body K

Ta         = The absolute temperature of the surroundings K

σ            = Stefan-Boltzmann Constant σ =5.67x10-8 wm2k4

This is the energy emitted from the surface.

At a distance x from the surface the energy received (and indicated) by a detector R will be related to the Stefan-Boltzmann constant by a factor F such that.

R=F x σTs4- Ta4

## Procedure

- Ensure that the H111 main switch is in the off position (the three digital displays should not be illuminated).

- Turn the voltage controller anti-clock wise to set the AC voltage to minimum.

- Install the heated plate at the left-hand side of the track and install the radiometer on the right-hand carriage. No items are installed in the left-hand carriage for the experiment but one of the black plates should be placed on the bench and connected to the thermocouple socket T9.

- Fit the light radiometer to the sensor carriage

- Ensure that the radiation shield is in position in the radiometer aperture and station the radiometer in the 900mm position

- The radiometer should be left for several minutes after handling with the radiation shield in position to ensure that residual heating has dissipated.

- For radiometer experiments, position the Wm2 displays console on top of the H111 console. Connect the power cable between both consoles and plug the radiometer signal cable into the front panel.

- Turn on the H111 main switch and three digital displays should illuminate. The radiometer should also illuminate. The required temperature is displayed on the LED digital display by turning the rotary sector switch.

- ‘Auto-Zero’ the radiometer by pressing the right hand * button twice.

###### Figure 4.1:  Schematic diagram of experiment

- Rotate the voltage controller clockwise to increase the voltage.

- Monitor the Wm2 digital displays and after several minutes, the display should reach a minimum.

- Finally, “Auto-Zero” the radiometer by pressing the right hand * button twice.

- Leave the radiation shield in position and rotate the voltage controller clockwise to increase the voltage to maximum volts. Select the T10 position on the temperature selector switch and monitor the T10 temperature.

- When the T10 temperature has reached a maximum condition, remove the radiation shield (without touching the radiometer). Immediately the indicated value should start to rise. Monitor the digital display until the displayed value reaches a maximum and then record the following.

T7, T10, X (900mm in this case), R.

- Again, without touching the radiometer, move the carriage holding the radiometer to a position 800mm from the heated plate. Again, the radiometer reading will start to rise. Allow this to reach a maximum and repeat the observations

T7, T10, X (800mm in this case), R.

- Repeat the above procedure in reducing steps of 100mm until the radiometer is 200mm from the heated plate.

## Observations:

Sample test results

For the unit tested C = 0.786

| V | I | T7 | T10 | x | R | Rc |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 900 |  |  |
|  |  |  |  | 800 |  |  |
|  |  |  |  | 700 |  |  |
|  |  |  |  | 600 |  |  |
|  |  |  |  | 500 |  |  |
|  |  |  |  | 400 |  |  |
|  |  |  |  | 300 |  |  |
|  |  |  |  | 200 |  |  |

Table 4.1: Measurement of temperatures from radiometer

While

T7= Temperature of black body

T10= Temperature of heater

X=Distance of radiometer from heater

V= Voltage

I= Current

R= Reading of radiometer in Wm2

Rc= Corrected radiometer reading=Rc=R × C

## Calculated Data

### Objective:01

To verify the inverse square law for thermal radiation

| Rc | x | Log10 x | Log10Rc |
| --- | --- | --- | --- |
|  | 900 |  |  |
|  | 800 |  |  |
|  | 700 |  |  |
|  | 600 |  |  |
|  | 500 |  |  |
|  | 400 |  |  |
|  | 300 |  |  |
|  | 200 |  |  |

##### Table 4.2: Calculation of intensity of radian

The data may either be converted to Log10 format as shown above and then plotted on a linear graph or alternatively if log-log graph paper is available, the data may be plotted directly without taking log values.

#### Graph

Plot a graph b/w log10x and log10Rc

#### Conclusion

### Objective:02

To verify the Stefan Boltzmann Law

| Ts | Ta | qb | Rc | F= Rcqb |
| --- | --- | --- | --- | --- |

##### Table 4.3: Calculation of shape factor

#### Specimen Calculation

For the first sample the calculations are as follows:

Ta = T9 + 273.15                           Ts = T10+273.15   While T9= ambient temperature

Hence                            qb= σ (Ts4 -Ta4 )

From the radiometer reading:

Rc= C x R    (Wm2)

Hence   F= Rcqb

It may be seen from the test results that the factor F remains essentially constant thereby demonstrating that the Steffen-Boltzmann relationship applies.

#### Conclusion

## Statistical Analysis

For Shape Factor F

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2

## Questions

- What is value of Log10 x corresponding to the value of 550 mm, Calculate from graph

- How does thermal radiation differ from other types of electromagnetic radiation?

- What is inverse square law for thermal radiation

- Define absorptivity, reflectivity and transmissivity

- What is the Stefan-Boltzmann law?

## Comments

# LAB SESSION 5

To verify the role of shape factor involved in radiation heat transfer

## Apparatus

Radiation Heat Transfer unit H111C (Serial no H111C/01310)

## Main Parts

- Heat Source

- Radiation detector

- Temperature Sensors

- Radiometer output unit

- Main digital Control panel (H111)

## Useful Data

Laws of radiant heat transfer and radiant heat exchange (H111C)

Stefen-Boltzman Constant σ =5.67x10-8 wm2k4

## Theory

### Radiation

Radiation is the emission or transmission of energy in the form of waves or particles through space or through a material medium.

### Shape Factor

F= Rσ (Ts4- Ta4

F= Rqb

It can be shown that the view factor F is related to the view angle θ such that

###### Figure.5.1:  Schematic diagram for shape factor

F=sin2θ

Hence

R= sin2θ X σ X(Ts4 -Ta4 )  = sin2θ X qb       Equation 5.1: Energy received by detector

Test data is shown overleaf to illustrate the relationship.

## Procedure

- Install the heated plate at the left-hand side of the track and install the radiometer on the right-hand carriage. No items are installed in the left-hand carriage for the experiment but one of the black plates should be placed on the bench and connected to the thermocouple socket T9

- Fit the light radiometer to the sensor carriage.

- Ensure that the radiation shield is in position in the radiometer aperture and station the radiometer in the 900mm position

- The radiometer should be left for several minutes after handling with the radiation shield in position to ensure that residual heating has dissipated.

- For radiometer experiments, position the Wm2 displays console on top of the H111 console. Connect the power cable between both consoles and plug the radiometer signal cable into the front panel.

- Turn on the H111 main switch and three digital displays should illuminate. The radiometer should also illuminate. The required temperature is displayed on the LED digital display by turning the rotary sector switch.

- ‘Auto-Zero’ the radiometer by pressing the right hand * button twice.

Figure 5.2:  Schematic diagram of experiment

- Rotate the voltage controller clockwise to increase the voltage.

- Monitor the Wm2 digital displays and after several minutes, the display should reach a minimum.

- Finally, “Auto-Zero” the radiometer by pressing the right hand * button twice.

- Leave the radiation shield in position and rotate the voltage controller clockwise to increase the voltage to maximum volts. Select the T10 position on the temperature selector switch and monitor the T10 temperature.

- When the T10 temperature has reached a maximum condition, remove the radiation shield (without touching the radiometer). Immediately the indicated value should start to rise. Monitor the digital display until the displayed value reaches a maximum and then record the following.

T7, T10, X (900mm in this case), R.

- Again, without touching the radiometer, move the carriage holding the radiometer to a position 800mm from the heated plate. Again, the radiometer reading will start to rise. Allow this to reach a maximum and repeat the observations

T7, T10, X (800mm in this case), R.

- Repeat the above procedure in reducing steps of 100mm until the radiometer is 200mm from the heated plate.

## Observations:

Sample test results

For the unit tested C = 0.786

| V | I | T7 | T10 | x | R | Rc |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 900 |  |  |
|  |  |  |  | 800 |  |  |
|  |  |  |  | 700 |  |  |
|  |  |  |  | 600 |  |  |
|  |  |  |  | 500 |  |  |
|  |  |  |  | 400 |  |  |
|  |  |  |  | 300 |  |  |
|  |  |  |  | 200 |  |  |

Table 5.1: Measurement of temperatures from radiometer

While

T7= Temperature of black body

T10= Temperature of heater

X=Distance of radiometer from heater

V= Voltage

I= Current

R= Reading of radiometer in Wm2

Rc= Corrected radiometer reading=Rc=R × C

### Calculated Data

| Ts | Ta | qb | θ | sin2θ | Rc= qb x sin2θ (Theoretical) | Rc (Experimental) |
| --- | --- | --- | --- | --- | --- | --- |
| K | K | Wm2 | Radians |  | Wm2 | Wm2 |

##### Table 5.2: Radiation incident on the detector

#### Specimen Calculation

For the first data point at x = 900mm

Ts=T10+273.15                                         Ta=T9+273.15

Hence                                            qb= σ (Ts4- Ta4)

From the geometry                             θ= tan-1(50900)

Hence                                                           sin2θ=

From this data                                                 Rc=qv x sin2θ qb. sin2θ

The corrected radiation Rc recorded by the radiometer under these conditions was

Rc =   (  )  Wm2

Comparing the calculated radiation incident on the detector qb x sin2θ and the corrected radiation measured by the detector Rc it may be seen that the value are similar.

Note that small errors in temperature measurement affect the data to the fourth power i.e. T4

#### Conclusion

## Statistical Analysis

NA

## Questions

- What is a gray body

- What is meant by the radiation shape factor?

- What is radiation shields and its effect

## Comments

# LAB SESSION 6

To calculate the total heat transfer of combined convection and radiation heat transfer mechanism

## Learning Objective

- Determination of the combined (radiation and convection) heat transfer (Qr +  Qc ) from a horizontal cylinder.

## Apparatus

Combined Convection and Radiation Heat Transfer unit H111D (Serial No= H111D/01641)

## Main Parts

- Heater Power

- T10 Heater surface temperature

- Air velocity sensor (Hot wire anemometer)

- T9 Air temperature

- Throttle butterfly

- Fan

- Main Switch

- Main digital Control panel (H111)

## Useful Data

Combined Convection and Radiation H111D

| T | ν | k | Pr |
| --- | --- | --- | --- |
| K | m2/s | w/mK | _ |
| 300 | 1.568E-05 | 0.02624 | 0.708 |
| 350 | 2.076 E-05 | 0.03003 | 0.697 |
| 400 | 2.590 E-05 | 0.03365 | 0.689 |
| 450 | 2.886 E-05 | 0.03707 | 0.683 |
| 500 | 3.790 E-05 | 0.04038 | 0.680 |
| 550 | 4.434 E-05 | 0.04360 | 0.680 |
| 600 | 5.134 E-05 | 0.05659 | 0.680 |

Table 6.1: Table of Physical properties of Air at Atmospheric Pressure

The above data is presented graphically on Graphs D1, D2, and D3.

If spreadsheet is to be utilized for data evaluation, then the values may be determined with reasonable accuracy from the following equations.

Where T is the air temperature in K

Cylinder diameter D = 0.01m

Cylinder heated length L = 0.07m

Cylinder effective heated area As = 0.0022m2

Effective air velocity local to cylinder due to blockage effect Ue = Ua X 1.22

## Theory

When a horizontal cylinder, with its surface at a temperature above that of its surroundings, is located in stationary air, the heat loss from the cylinder will be a combination of natural convection to the air (air surrounding the cylinder becomes less dense and rises when it is heated) and radiation to the surroundings.

Heat loss due to conduction is minimized by the design of the equipment and measurements mid-way along the heated section of the cylinder can be assumed to be unaffected by conduction at the ends of the cylinder. Heat loss by conduction would normally be included in the analysis of a real application.

The following theoretical analysis uses an empirical relationship for the heat transfer due to natural convection proposed by WH McAdams in the publication "Heat Transmission", third edition, McGraw-Hill, New York, 1959.

Total heat loss from the cylinder: Qtotal = Qc + Qr

Equation 6.1

Heat loss due to natural convection: Qc = hc As (Ts - Ta)

Equation 6.2

Heat loss due to radiation: Qr = hr As (Ts - Ta)

Equation 6.3

Heat transfer area (surface area): As = (π D L)

Equation 6.4

The heat transfer coefficients hc and hr can be calculated using the following relationships

hc=1.32 Ts-TaD0.25                                                     Equation 6.5

hr=ϵϭ Ts4-Ta4Ts-Ta                                                             Equation 6.6

ϭ= Stefan Boltzmann constant

Є = Emissivity of surface (dimensionless)

Ts = Surface temperature of cylinder (K)

Ta = Ambient temperature (K)

## Procedure

- Turn the voltage controller anti-clock wise to set the AC voltage to minimum. Ensure the Combined Convection and Radiation H111D accessory has been connected to the Heat Transfer Service Unit H111.

- Ensure that the heated cylinder is located in its holder at the top of the duct and that the cylinder is rotated so that the thermocouple location is on the side of the cylinder.

- Turn on the main switch and the digital displays should illuminate. Turn the rotary selector switch to display T10. Rotate the voltage control clockwise to increase the voltage. Note that for natural convection experiments are to be undertaken it is recommended that the cylinder surface temperature T10 is NOT allowed to exceed 500℃.

- Rotate the voltage controller to give a 50-volt reading.

- Select the temperature position T10 using the rotary selector switch and monitor the temperature.

- Open the throttle butterfly on the fan intake but do not turn on the fan switch, as the fan will not be used for natural convection.

- When T10 has reached a steady state, temperature record the following T9, T10, V, I.

- Increase the voltage controller to give an 80-volt reading, monitor T10 for stability and repeat the readings.

- Increase the voltage controller to give a 120-volt reading, monitor T10 for stability and repeat the readings.

- Increase the voltage controller to give a 150-volt reading, monitor T10 for stability and repeat the readings for different voltages

Figure 6.1:  Schematic Diagram of Experiment

## Calculated Data

### Objective:01

Determination of the combined (radiation and convection) heat transfer (Qr + Qc ) from a horizontal cylinder.

#### Observations

Sample test results

| Sample | V | I | T9 | T10 |
| --- | --- | --- | --- | --- |
| - | Volts | Amps | ℃ | ℃ |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

##### Table 6.2: Measurement of temperature for free convection

T9= Ambient Temperature

T10= Heater temperature

V= Input voltage

I= Input current

| Sample | Qinput | hr | hc | Qr | Qc | Qtot |
| --- | --- | --- | --- | --- | --- | --- |
| - | W | Wm2k | Wm2k | W | W | W |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |

##### Table 6.3: Heat transfer due to free convection and radiation

#### Specimen Calculations

For the first sample the calculation are as follows:

Qinput  =V I
For the radiant component;

hr= εF σ(Ts4- Ta4)(Ts-Ta)

Ts = T 10+ 273.15 (K)

Ta= T9 + 273.15 (K)

hr=0.95 X 1 X 5.67 X 10-8 (Ts4- Ta4)(Ts-Ta)

=               Wm2k

Hence the heat lost to radiation Qr

(As is obtained from USEFUL DATA on)

Qr = hrAs (Ts-Ta)

For the convective component using the simple formula

hc=1.32 Ts-TaD0.25

=     Wm2k

Hence the heat lost to convection;

(As Is obtained from USEFUL DATA)

Qc = hcAs (Ts-Ta)

Hence the total heat lost by calculation

Qtot= Qr+Qc  =         (W)

In addition it may be seen that at low temperatures the convective component of heat transfer is predominant while at higher temperatures the radiant component becomes predominant.

The temperature, at which the conditions reverse, is also influenced by the emissivity of the surface, and that of the surroundings.

#### Graph

Draw the Graph of hc, hr (y-Axis) and surface Temperature T10(X- Axis)

#### Conclusion

## Statistical Analysis

For total heat transfer in free convection

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Questions

- Write about the trend of convective heat transfer coefficient with heater temperature

- What is value of heater temperature at air velocity of 2.3 m/s, compute it from graph.

- Write about the trend of Radiant heat transfer coefficient with heater temperature

## Comments

# LAB SESSION 7

To calculate the total heat transfer of combined “forced convection” and “radiation” heat transfer mechanism

## Learning Objective

- Determination of the effect of forced convection on the heat transfer from a cylinder at varying air velocities.

## Apparatus

Combined Convection and Radiation Heat Transfer unit H111D (Serial No= H111D/01641)

## Main Parts

- Heater Power

- T10 Heater surface temperature

- Air velocity sensor (Hot wire anemometer)

- T9 Air temperature

- Throttle butterfly

- Fan

- Main Switch

- Main digital Control panel (H111)

## Useful Data

Combined Convection and Radiation H111D

| T | ν | k | Pr |
| --- | --- | --- | --- |
| K | m2/s | w/mK | _ |
| 300 | 1.568E-05 | 0.02624 | 0.708 |
| 350 | 2.076 E-05 | 0.03003 | 0.697 |
| 400 | 2.590 E-05 | 0.03365 | 0.689 |
| 450 | 2.886 E-05 | 0.03707 | 0.683 |
| 500 | 3.790 E-05 | 0.04038 | 0.680 |
| 550 | 4.434 E-05 | 0.04360 | 0.680 |
| 600 | 5.134 E-05 | 0.05659 | 0.680 |

Table 7.1: Table of Physical properties of Air at Atmospheric Pressure

The above data is presented graphically on Graphs D1, D2, and D3.

If spreadsheet is to be utilized for data evaluation then the values may be determined with reasonable accuracy from the following equations.

Where T is the air temperature in K

Cylinder diameter D = 0.01m

Cylinder heated length L = 0.07m

Cylinder effective heated area As = 0.0022m2

Effective air velocity local to cylinder due to blockage effect Ue = Ua X 1.22

## Theory

### Forced convection

Forced convection is a mechanism, or type of transport in which fluid motion is generated by an external source (like a pump, fan, suction device, etc.). It should be considered as one of the main methods of useful heat transfer as significant amounts of heat energy can be transported very efficiently.

Actual power supplied to the heated cylinder Qin = V I (Watts)	             Equation 7.1

Qr = hr As (Ts - Ta)

So Nusselt number is

Nu=0.3+[{(0.62Re0.5Pr0.33)/(1+(0.4Pr)0.66 )0.25 } *(1+ (Re282000)0.5) ]        Equation 7.2

And Reynolds number is

Re=UeD/v                                                                 Equation 7.3

hf=kDNu                                                                                  Equation 7.4

This results in a convective heat transfer of

Qf=hfAs(Ts-Ta)                                                                           Equation 7.5

Hence the total heat transfer from the cylinder

Qtotal=(Qr+Qf )                                                                        Equation 7.6

## Procedure

- Ensure that the heated cylinder is located in its holder at the top of the duct and that the cylinder is rotated so that the thermocouple location is on the side of the cylinder. This is shown schematically below.

- Turn on the main switch and the digital displays should illuminate. Turn the rotary selector switch to display T10. Rotate the voltage control clockwise to increase the voltage.

- Rotate the voltage controller to give a 50-volt reading.

- Select the temperature position T10 using the rotary selector switch and monitor the temperature.

- Open the throttle butterfly on the fan intake and turn on the fan switch,

- When T10 has reached a steady state, temperature record the following T9, T10, V, I.

- Increase the voltage controller to give an 80-volt reading, monitor T10 for stability and repeat the readings.

- Increase the voltage controller to give a 120-volt reading, monitor T10 for stability and repeat the readings.

- Increase the voltage controller to give a 150-volt reading, monitor T10 for stability and repeat the readings.

- Finally increase the voltage controller to give approximately a 185-volt reading, monitor T10 for stability and repeat the readings. However, the temperature of the cylinder should not be allowed to exceed 500℃ and if local conditions result in a higher temperature the voltage should be reduced accordingly.

Figure 7.1:  Schematic Diagram of Experiment

### Objective

Determination of the effect of forced convection on the heat transfer from a cylinder at varying air velocities.

#### Observations

Sample test results

| Sample | V | I | Ua | T9 | T10 |
| --- | --- | --- | --- | --- | --- |
| - | Volts | Amps |  |  |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

##### Table 7.2: Measurement of temperature due to forced convection

T9= Ambient Temperature

T10= Heater temperature

V= Input voltage

I= Input current

#### Calculated Data

Calculation results in the following parameters

| Sample | Qinput | Ue | Pr | v | k | Re | Nu |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - | W | m/s |  | m2/s | W/mK | - | - |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |

| Sample | hr | Qr | hf | Qf | Qtotal |
| --- | --- | --- | --- | --- | --- |
| - | W/m2K | W | W/m2K | W | W |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

##### Table 7.3: Heat transfer due to forced convection

#### Specimen Calculations

For the first sample the calculations are as follows

Qinput=V*I      (W)

For the radiant component

hr= εF σ(Ts4- Ta4)(Ts-Ta)

Ts = T 10+ 273.15 (K)

Ta= T9 + 273.15 (K)

hr=0.95 × 1 X 5.67 × 10-8 (Ts4- Ta4)(Ts-Ta)

=               Wm2k

This results in a radiant heat transfer of

Qr = hrAs (Ts-Ta)

For the above convective component using the formula

Nu=0.3+{[(0.62Re0.5Pr0.33)/(1+(0.4Pr)0.66 )0.25]  ×(1+ Re282000)0.5}

First the physical parameters must be determined at the air stream temperature T9

From the graphs D1, D2, D3 for example at T9=296K

v=1.4*10-5 m2/s

K=0.026 W/mK

Pr=0.716

The measured duct velocity Ua is locally increased around the cylinder to Ue (effective air velocity) due to the blockage effect of the cylinder itself. This relates to the area ratio between the duct cross sectional area and the plan area of the cylinder in the duct.

Ue=Ua*1.22   (m/s)

Hence                                                     Re=UeD/v

Hence substituting the values in the equation;

Nu =0.3+(0.62Re0.5Pr0.33)/(1+(0.4Pr)0.66 )0.25  *(1+ (Re282000)0.5)

From the Nusselt number Nu ;

hf=kDNu                 (Wm2k)

This results in a convective heat transfer of

Qf=hfAsTs-Ta

Hence the total heat transfer from the cylinder                                                                                               Qtotal=(Qr+Qf )

#### Graph:

If the surface temperature T10 is plotted against the effective air velocity Ue it may be seen that for a constant heat input the surface temperature falls as the velocity increases.

#### Conclusion

## Statistical Analysis

For total heat transfer in forced convection

xavg= x1+x2+x3n

Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Questions

- What is free and forced convection

- What is drag coefficient

## Comments

Graph 1

Graph 7.1: Kinematic velocity of air at standard Pressure

GRAPH# D2

Graph 7.2: Thermal conductivity of air at standard pressure

GRAPH# D3

Graph 7.3:  Prandtl number of air at standard pressure

# LAB SESSION 8

To experimentally verify the heat transfer from an extended surface from combined modes of free conduction, free convection and radiation heat transfer by comparing it with the theoretical analysis

## Learning Objective

To analyse the increase in heat transfer by using extended surface body

## Apparatus

Extended Surface Heat Transfer unit H111E (Serial no H111E/00832)

## Main Parts

- Heated cylinder

- Eight temperature sensors (Thermocouples)

- Heater

- Heat transfer service unit (H111)

## Useful Data

- HEATED ROD Diameter     D= 0.01m

- Heated rod length                 L=    0.35m

- Heated rod effective cross-sectional area              As=7.854×10-5m2

- Heated rod surface area                                          A=0.01099m2

- Thermal conductivity of heated rod material         k=121  Wm2

- Stefan Boltzmann constant                                     σ   =   5.67 × 10-8W/m2

## Theory:

### Fin (extended surface)

In the study of heat transfer, fins are surfaces that extend from an object to increase the rate of heat transfer to or from the environment by increasing convection. The amount of conduction, convection, or radiation of an object determines the amount of heat it transfers. Increasing the temperature gradient between the object and the environment, increasing the convection heat transfer coefficient, or increasing the surface area of the object increases the heat transfer. Sometimes it is not feasible or economical to change the first two options. Thus, adding a fin to an object, increases the surface area and can sometimes be an economical solution to heat transfer problems.

The heat Transferred can be calculated at a given point x

Qx=kAmTx-TatanhmL               Equation 8.1: Heat transferred through extended surface

While

Qx=Heat transferred at a given point x

Tx=Temperature at a given point x

Ta= Ambient temperature

L= Length of rod

m=hPkA

h=Overall heat transfer coefficient

P= Perimeter

K= Thermal conductivity

A= Heated rod surface area

## Procedure:

- Turn the voltage controller anti-clock wise to set the AC voltage to minimum. Ensure the Extended Surface Heat Transfer Unit H111E accessory has been connected to the Heat Transfer Service Unit H111.

- After adjusting the heater voltage ensure that T1 (the thermocouple closest to the heater) varies in accordance with the sense of adjustment. i.e. if the voltage is increased the temperature T1 should also increase, if the voltage is reduced the temperature T1 should be reduce.

- It is now necessary that to monitor the temperature T1 to T8 until all the temperatures are stable.

- When T1 through T8 have reached a steady state, temperatures record the following. T1 to T9, V and I.

- If time permits increase the voltage to a 120volts reading, repeat the monitoring of all temperatures and when stable repeat the above readings.

- When the experimental procedure is completed, it is good practice to turn off the power to the heater by reducing the AC voltage to zero and leaving the fan running for a short period until the heated cylinder has cooled. Then turn off the main switch.

Figure 8.1:  Schematic Diagram of Experiment

## Observations

| Sample No |  | 1 | Distance From T1(m) |
| --- | --- | --- | --- |
| V | volts | - | - |
| I | Amps | - | - |
| t1 | ℃ |  | 0 |
| t2 | ℃ |  | 0.05 |
| t3 | ℃ |  | 0.1 |
| t4 | ℃ |  | 0.15 |
| t5 | ℃ |  | 0.2 |
| t6 | ℃ |  | 0.25 |
| t7 | ℃ |  | 0.3 |
| t8 | ℃ |  | 0.35 |
| t9 | ℃ |  | - |

Table 8.1: Temperatures measured at different distances from T1

## Calculated Data

| Heat input Qin W | Distance x form T1 (m) | Tx-TaT1-Ta | cos h[m(L-x)]cosh [m L] | Calculated heat transferred |
| --- | --- | --- | --- | --- |
|  | 0 |  |  |  |
|  | 0.05 |  |  |  |
|  | 0.1 |  |  |  |
|  | 0.15 |  |  |  |
|  | 0.2 |  |  |  |
|  | 0.25 |  |  |  |
|  | 0.3 |  |  |  |
|  | 0.35 |  |  |  |

Table 8.2: Heat transfer from extended surface

### Specimen Calculation

For the first sample the calculations are as follows

Qinput=V*I      (W)

The value of h will be that due to both convective and radiation heat transfer

h= hr+hc

For the radiant component;

hr=εFσTmean4-Ta4Tmean-Ta =  Wm2k

Tmean= t1+t2+t3+t4+t5+t6+t7+t88+273.15

Ta = T9 + 273.15 K

Where σ= Stefan Boltzmann Constant σ = 5.67 × 10-8Wm2 K

F = Shape factor or view factor (relating to the element geometry and the surroundings)

ε = Emissivity of the rod surface = 0.95

Ta  = Absolute ambient temperature t9 + 273.15K

Tmean = Absolute mean of the measured surface temperature of the rod.

Tmean= t1+t2+t3+t4+t5+t6+t7+t88+273.15

For the convective component;

hc=1.32 Tmean-TaD0.25=(Wm2k)

This is a simplified empirical equation for natural convection from a horizontal cylinder. Where

Ta       = Absolute ambient temperature t9 + 273.15K

Tmean= Absolute mean of the measured surface temperature of the rod.

D        = Diameter of the rod

Hence the overall estimated heat transfer coefficient

h= hr+hc =    (Wm2 K)

Now m can be determined to a reasonable degree of accuracy such that						m=hPkA

Where P is the perimeter of the rod = π D

Now the heat Transferred can be calculated at a given point x;

Qx=kAmTx-TatanhmL

While	Tan h (mL) =emL -e-mLemL+e-mL

And                                                         Cos h (mL)= emL +e-mL2

Note: For insulated fin tip and negligible heat loss from fin tip
cos h [ mL-x]cosh   [m L] Should be equal to  Tx-TaT1-Ta

### Graph

Draw the graph b/w the distance from T1 thermocouple (X-Axis) and Temperature (Y-Axis).

### Conclusion

## Questions

- How heat transfer is varied by varying heat transfer area

- What is effect of perimeter on heat transfer

- At which position, the temperature reaches the maximum value, give reason.

## Comments

# LAB SESSION 9

# To Reduce radiation errors in measurement of temperature by using shield between sensor and source of radiation

## Learning Objective

- To reduce the errors in temperature measurement due to radiation from a source by using radiation shield between the sensor and the source of radiation.

## Apparatus

Radiation Errors in temperature Measurements H111F (Serial no H111F/00315)

## Main Parts

- Radiant Source

- Temperature Sensors

- Shields of different grades

- Main digital Control panel (H111)

## Theory

The objective for the temperature sensors is to accurately measure the temperature of the air stream. In order to do this the sensors must stabilize at the same temperatures as the air stream. Under ideal conditions the sensors will gain or lose heat until stabilization occurs and the sensors are at the same temperature as the air. However, if a source of thermal radiation is visible to the sensor then depending upon the emissivity of the sensor and the source more or less energy may be absorbed by the sensor. Hence the sensor may stabilize at a temperature above or below the temperature of the air depending upon the temperature of the radiant source.

The magnitude of the difference in sensor temperature relative to the air stream will depended upon several factors.

- The difference in temperature between the sensor and the radiating surface.

- The velocity of the air passing the sensor.

- The size of the sensor.

- The emissivity of the sensor, the surroundings and the radiant source.

- The thermal conductivity of the sensor material and its connecting lead.

- Other material that may be in the air stream such as dust or water vapor.

If a suitable shield can be placed between the radiant source and the thermocouple the errors in measurement can be reduced. Though the shield will be heated or cooled by the radiant source the temperature reached by the shield will have less influence on the thermocouple then that of the radiant source. In the apparatus, the shield is a polished stainless steel plate that can be lowered between the ceramic heater and the three test thermocouples. The shield allows air to pass through but prevents direct line of sight influence from the radiant source

## Procedure:

- Turn the voltage controller anti-clock wise to set the AC voltage to minimum. Ensure the Radiation Errors H111F accessory has been connected to the Heat Transfer Service Unit H111.

- Ensure that the radiation shield is not fitted.

- Turn on the main switch and the digital displays should illuminate. Select the temperature position T10 using the rotator switch and monitor the temperatures regularly.

- Set the temperature selector switch to display temperatures T6, T7, T8, T9 and T10 and record the values.

- Open the throttle butterfly but do not turn on the centrifugal fan at this point.

- Rotate the voltage controller clockwise to increase the heater voltage to 80 Volts. Select the T10 position on the temperature selector switch and monitor the T10 temperature. Also monitor temperatures T7 to T9 until these reach a stable temperature (Note that the temperatures may not be equal).

- When the temperatures are stable record T6, T7, T8, T9, T10, V, and I.

- Rotate the voltage controller clockwise to increase the heater voltage to 120 Volts. Again allow the temperatures to stabilize and then record T6, T7, T8, T9, T10, V, and I.

- Repeat the above procedure at heater voltages of 180 and 230 volts (Note that the heater temperature T10 should not be allowed to exceed 400℃).

- Completely close the throttle butterfly on the fan intake and then turn on the centrifugal fan.

- Adjust the throttle until a velocity of approximately 0.5m/s is indicated.

- Observe the temperature T6 to T10 and when stable record T6, T7, T8, T9, T10, V, and I.

- Increase the air velocity to 1m/s, allow the temperatures to stabilize and repeat the observations. Further increase the velocity to 2m/s and 4m/s and again repeat the observations.

- Repeat all the procedure for objective 2 by using the radiation shield.

## Observations

### Objective

To demonstrate how temperature measurements can be affected by radiant heat transfer to a sensor from its surroundings

|  |  | WITHOUT FAN ASSISTED AIR FLOW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | V | I |  | Ua | T6 | T7 | T8 | T9 | T10 |
| 1 |  |  | No Shield | 0 |  |  |  |  |  |
|  |  |  | Shield | 0 |  |  |  |  |  |
| 2 |  |  | No Shield | 0 |  |  |  |  |  |
|  |  |  | Shield | 0 |  |  |  |  |  |
| 3 |  |  | No Shield | 0 |  |  |  |  |  |
|  |  |  | Shield | 0 |  |  |  |  |  |
| 4 |  |  | No Shield | 0 |  |  |  |  |  |
|  |  |  | Shield | 0 |  |  |  |  |  |
| 5 |  |  | No Shield | 0 |  |  |  |  |  |
|  |  |  | Shield | 0 |  |  |  |  |  |
|  |  | WITH FAN ASSISTED AIR FLOW |  |  |  |  |  |  |  |
| 1 |  |  | No Shield |  |  |  |  |  |  |
|  |  |  | Shield |  |  |  |  |  |  |
| 2 |  |  | No Shield |  |  |  |  |  |  |
|  |  |  | Shield |  |  |  |  |  |  |
| 3 |  |  | No Shield |  |  |  |  |  |  |
|  |  |  | Shield |  |  |  |  |  |  |
| 4 |  |  | No Shield |  |  |  |  |  |  |
|  |  |  | Shield |  |  |  |  |  |  |

##### Table 9.1: Temperatures at different points from radiant source

While

V= Voltage

I= Current, T6= Temperature of air and T7, T8, T9= Temperature at different points

T10= Temperature of heater

Ua= Air velocity

#### Graph

- Draw the graph b/w heater volts (V) along x-Axis and Temperature T6, T7, T8, T9 along Y-Axis. (i.e. V vs T6, V vs T7, V vs T8, V vs T9, V vs T10) with shield and without shield

#### Conclusion

## Comments

# LAB SESSION 10

To determine the Convective heat transfer coefficient of a solid cylinder using analytical transient temperature/heat flow charts

## Learning Objective

- Use of heisler charts for lumped heat capacity system

- Use of properties like biot number and fourier number to calculate convective heat transfer unit

## Apparatus

Heat Transfer service unit with Unsteady State Heat Transfer Unit H111G

## Main Parts

- Drain Valve

- Water supply

- Pump

- 20mm diameter Brass cylinder

- Temperature sensors prob

- water bath

- 20 mm diameter stainless steel cylinder

## Theory

### Transient Heat-Transfer

If a solid body is suddenly subjected to a change in environment, some time must elapse before an equilibrium temperature condition will prevail in the body.  In the transient heating or cooling process that takes place in the interim period before equilibrium is established, the analysis must be modified to take into account the change in internal energy of the body with time, and the boundary conditions must be adjusted to match the physical situation that is apparent in the unsteady-state heat-transfer problem. Unsteady-state heat-transfer analysis is obviously of significant practical interest because of the large number of heating and cooling processes that must be calculated in industrial applications

### Lumped-Heat-Capacity System

Transient heat conduction can be analyzed in systems that may be considered uniform in temperature. This type of analysis is called the lumped-heat-capacity method. Such systems are obviously idealized because a temperature gradient must exist in a material if heat is to be conducted into or out of the material. In general, the smaller the physical size of the body, the more realistic the assumption of a uniform temperature throughout; in the limit a differential volume could be employed as in the derivation of the general heat-conduction equation

### Biot number (Bi)

The Biot number (Bi) is a dimensionless quantity used in heat transfer calculations. It is named after the French physicist Jean-Baptiste Biot (1774–1862), and gives a simple index of the ratio of the heat transfer resistances inside of and at the surface of a body. This ratio determines whether or not the temperatures inside a body will vary significantly in space, while the body heats or cools over time, from a thermal gradient applied to its surface. In general, problems involving small Biot numbers (much smaller than 1) are thermally simple, due to uniform temperature fields inside the body. Biot numbers much larger than 1 signal more difficult problems due to non-uniformity of temperature fields within the object. It should not be confused with Nusselt number, which employs the thermal conductivity of the fluid and hence is a comparative measure of conduction and convection, both in the fluid.The Biot number has a variety of applications, including transient heat transfer and use in extended surface heat transfer calculations.

The Biot number is defined as

Bi=hLk                               Equation 10.1: Biot number

h = film coefficient or heat transfer coefficient or convective heat transfer coefficient

L = characteristic length, which is commonly defined as the volume of the body divided by the surface area of the body, such that                                        L=VolumeArea

k = Thermal conductivity of the body

The physical significance of Biot number can be understood by imagining the heat flow from a small hot metal sphere suddenly immersed in a pool, to the surrounding fluid. The heat flow experiences two resistances: the first within the solid metal (which is influenced by both the size and composition of the sphere), and the second at the surface of the sphere. If the thermal resistance of the fluid/sphere interface exceeds that thermal resistance offered by the interior of the metal sphere, the Biot number will be less than one. For systems where it is much less than one, the interior of the sphere may be presumed always to have the same temperature, although this temperature may be changing, as heat passes into the sphere from the surface. The equation to describe this change in (relatively uniform) temperature inside the object is simple exponential one described in Newton's law of cooling.

In contrast, the metal sphere may be large, causing the characteristic length to increase to the point that the Biot number is larger than one. Now, thermal gradients within the sphere become important, even though the sphere material is a good conductor. Equivalently, if the sphere is made of a thermally insulating (poorly conductive) material, such as wood, the interior resistance to heat flow will exceed that of the fluid/sphere boundary, even with a much smaller sphere. In this case, again, the Biot number will be greater than one.

### Fourier number (Fo)

Fourier number (Fo) or Fourier modulus, named after Joseph Fourier, is a dimensionless number that characterizes transient heat conduction. Conceptually, it is the ratio of diffusive or conductive transport rate to the quantity storage rate, where the quantity may be either heat (thermal energy) or matter (particles). The number derives from non-dimensionalization of the heat equation (also known as Fourier's Law) or Fick's second law and is used along with the Biot number to analyze time dependent transport phenomena.

The general Fourier number is defined as

Fo=Diffusive transport rateStorage rate

Fo = α X t(Length)2                        Equation 10.2: Fourier number

α=kρc                                       Equation 10.3: Thermal diffusivity

α is the thermal diffusivity (SI units: m2/s)

t is the characteristic time (s)

L is the length through which conduction occurs (m)

K is thermal conductivity (W/(m·K))

ρ is density (kg/m³)

c is specific heat capacity (J/(kg·K))

### Heisler charts

Heisler charts are a graphical analysis tool for the evaluation of heat transfer in thermal engineering. They are a set of two charts per included geometry introduced in 1947 by M. P. Heisler. which were supplemented by a third chart per geometry in 1961 by H. Gröber. Heisler charts permit evaluation of the central temperature for transient heat conduction through an infinitely long plane wall of thickness 2L, an infinitely long cylinder of radius ro, and a sphere of radius ro.

Although Heisler-Gröber charts are a faster and simpler alternative to the exact solutions of these problems, there are some limitations. First, the body must be at uniform temperature initially. Additionally, the temperature of the surroundings and the convective heat transfer coefficient must remain constant and uniform. Also, there must be no heat generation from the body itself.

## Procedure

- Ensure that the residual current circuit breaker (RCCB) is open-circuit. Ensure that the drain valve adjacent to the circulating pump is in the closed position and half-fill the water bath with clean water.

- Pump bleeding- Switch on the RCCB/MCB to cause the pump to run. Incline the pump by lifting the baseboard from the front to allow the air to escape. Noise from the pump is the sign of trapped air. A bleed screw is fitted to the head of the pump.

- Continue the filling of bath until the water level is at mid height of the holes in the flow duct. If the local water contains of a large amount of dissolved solids that normally result in scale build up then it is recommended that the bath id fitted with de-ionized or mineralized water. Ensue that the thermostat has been turned fully anti-clockwise and is in the off position.

- Ensure that the H111 unit main switch is in the off position (None of three digital displays should be illustrated). Ensure that the residual current circuit breaker on the H111 rear panel is in the ON position. Ensure that the residual current circuit breaker on the H111G baseboard is in the on position. Not that the residual current circuit breaker on the both units (H111 and H111G) should be tested for normal operation at intervals specified by local regulations.

- Turn on the power supply to the Unsteady State Heat Transfer unit and turn on the 16A Heater miniature circuit breaker (MCB). Ensure that the red power indicator adjacent to the thermostat is illustrated. Turn the thermostat to position 6 for faster heating. The water will take approximately 30 minutes to heat from cold. At this setting the water will boil, if left unattended. While the water bath is heating the following may be auctioned.

- Install the 20mm diameter brass cylinder in the shape carrier.

- Insert the T3 probe to engage fully into the center of the shape.

- Insert the T2 probe to sense water temperature adjacent to the shape.

- Avoid touching the shape by hand to reduce thermal effects and place the shape on the bench to reach ambient temperature.

- The water bath temperature T1 should be stabilized at approximately 80 to 90˚C

- Set the circulating pump to 3 and therefore the water flow velocity in the flow duct.

- Turn on the power supply to the Heat Transfer Service Unit H111 and main switch and three digital displays illustrate. Set the temperature selector switch to the T1to indicate the temperature of the bath. Observe the T1 to confirm that it is slowly increasing as the bath is heated.

- Record the start conditions temperatures and then plunge the shapes in the flow duct. Then record temperatures and time.

- Observe the T1. If the bath temperature exceeds that is required, reduce the thermostat setting to OFF and wait for the water to cool. The water bath temperature T1 should not be allowed to exceed 85-90 as the pump will cavitate.

- Once the 20mm diameter brass has reached the water bath temperature, remove it from the tank install the 20mm stainless steel cylinder in the shape carrier.

- Record the starting condition temperatures and then plunge the shape in the flow duct. Then record temperatures and time.

- Having achieved the desired temperature, say85, reduces the thermometer setting to position 2. It will cycle ON/OFF to maintain the existing temperature.

- If time permits the procedures may be repeated for the brass and stainless-steel sphere and/or the Brass and Stainless Slab.

- In addition, by varying the circulating pump speed, the effect of variation of water velocity on local heat transfer coefficient may be investigated.

- When the experimental procedure is completed, it is good habit to turn off the power to the heater by reducing the AC voltage to zero. And fan leaving the fan running for a short period until the heater has cooled. Then turn off the main switch.

## Observations

| Recorded Time | T1 Bath Temp. | T2 Air/ Water Temp. | T3 Geometric Centre Temp. |
| --- | --- | --- | --- |
| Seconds | ºC | ºC | ºC |
| 0 |  |  |  |
| 5 |  |  |  |
| 10 |  |  |  |
| 15 |  |  |  |
| 20 |  |  |  |
| 25 |  |  |  |
| 30 |  |  |  |
| 35 |  |  |  |
| 40 |  |  |  |

Table 10.1: Specimen: 20mm diameter Brass Cylinder.

| Recorded Time | T1 Bath Temp. | T2 Air/ Water Temp. | T3 Geometric Centre Temp. |
| --- | --- | --- | --- |
| Seconds | ºC | ºC | ºC |
| 0 |  |  |  |
| 5 |  |  |  |
| 10 |  |  |  |
| 15 |  |  |  |
| 20 |  |  |  |
| 25 |  |  |  |
| 30 |  |  |  |
| 35 |  |  |  |
| 40 |  |  |  |

Table 10.2: Specimen: 20mm Stainless Steel Cylinder

## Calculated Data

| Recorded Time | T1 Bath Temperature | T3 Geometric Centre Temperature | θ Non-dimensional Temperature | Fo Fourier Number | 1/Bi Inverse Biot Number | (h) Convective heat transfer coefficient |
| --- | --- | --- | --- | --- | --- | --- |
| Seconds |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |

Table 10.3:  Properties of Specimen: 20mm diameter Brass Cylinder

| Recorded Time | T1 Bath Temperature | T3 Geometric Centre Temperature | θ Non-dimensional Temperature | Fo Fourier Number | 1/Bi Inverse Biot Number | (h) Convective heat transfer coefficient |
| --- | --- | --- | --- | --- | --- | --- |
| Seconds |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |

Table 10.4:  Properties of Specimen: 20mm diameter stainless steel Cylinder

### Sample Calculations

The calculation procedure for system with finite internal and surface heat transfer resistance is as follows.

For each sample after intermission the non-dimensional temperature θ is calculated.

Tc=T3

T∞=T1

Ti=T3 at time 0

θ=Tc- T∞Ti- T∞=

Similarly, the Fourier Number For non-dimensional time

Fo = α× t(Length)2

Thermal diffusivity =α

α= kρc

For brass from USEFUL DATA

K=121 Wm-1K-1

ρ=8500 kgm-3

C=385 Jkg-1

α= k/ρc=1218500×385=3.7 X10-5

T =5 second

For cylinder the characteristics length=Length =        R = 10 X 10-3 m

Hence

Fo = αt(Length)2= 3.7 X10-5  X 5(10 X 10-3 )2=1.85

From the Heisler chart for a semi-infinite cylinder.

The co-ordinates for Fo =      and θ=       give

1Bi=

Where                                                       Bi=h x Lengthk

Length =          R the radius = 10×10-3m

Hence for the brass cylinder where                          k=121Wm-1k-1

h=(Bi x k ) R

This is the heat transfer coefficient around the cylinder due to the velocity of the hot water in the flow duct. This velocity depends only upon the pump speed.

For the stainless-steel cylinder

Repeat same calculations for stainless steel cylinder

### Graph

Draw graph between time (x-axis) and temperatures (y-axis)

### Conclusion

## Statistical Analysis

NIL

## Questions

- How the temperature is distributed at different interval of time

- What is Fourier Number

- What is Biot number

- What is Transient Heat-Transfer

- What is Lumped-Heat-Capacity System

- How Geometric Centre Temp is distributed at different intervals of time

- How Fourier number and Biot number vary with time. Give reason

## Comments

Graph 10.1: Heisler charts

# LAB SESSION 11

To measure the effect of different exchanger geometry on convection heat transfer mechanism

## Learning Objective

- Measure temperatures of different points in Free and forced convection in flat surfaces

- Measure the dependence of the heat transmission on exchanger geometry.

## Apparatus

Free and forced convection heat transfer unit TCLFC (Serial No= TCLFC 0009/07)

## Main Parts

- Stainless steel tunnel of rectangular section

- Flat exchanger, tenon exchanger and fins exchanger

- Thermocouple

- fan supply

- A branded computer attached to unit

- Interface

## Useful Data

- Structure of anodized aluminium that guarantees a good stability and resistance to the environment.

- Stainless steel tunnel 700 mm long, painted and resistant to corrosion.

- Methacrylate viewer that allows a good visualization of the exchanger that is in use.

- Grey PVC stabilizers to guarantee a uniform air flux.

- Thermocouples type J.

- Maximum working temperature: 100ºC.

- Flow sensor with digital output.

- Aluminium exchangers of flat, spiked and bladed surfaces.

- Heating resistances of 150W.

Dimensions and weights

- Approximate dimensions of the equipment: 37x61x92

- Approx. shipment volume: 0.2m3

- Approx. weight: 10 Kg

## Theory

Convection is the term used for the process of energy transmission, which takes place in the fluid mainly due to the energy transported by the movement of the fluid. The conduction energy process by molecular exchange is, as logical, always present. However, big quantities of energy (or heat) of the fluids are in contact with regions of inferior energy (colder regions) due to the existence of big displacements of the fluid particles. If external forces in form of difference of pressures produce the movement of the fluid, this mechanism is called forced convection. An example of forced convection is the pumping of a fluid, in contact with solid surfaces at different temperatures to that of the fluid. If no external forces are applied to a fluid, it can move due to density differences that could be produced by a solid body submerged in a fluid whose temperature is different to that of the fluid. This type of heat exchange is called free

Convection and it can be observed in a recipient with boiling water, in the air that surrounds radiators in rooms, etc. The following theoretical analysis uses an empirical relationship for the heat transfer due to natural convection proposed by WH McAdams in the publication "Heat Transmission", third edition, McGraw-Hill, New York, 1959

Heat loss due to natural convection: Qc = hc As (Ts - Ta)                                                          Equation 8.1

Heat transfer area (surface area): As = (π D L)                                                                         Equation 8.2

The heat transfer coefficient hc can be calculated using the following relationships

hc=1.32 Ts-TaD0.25                                                     Equation 8.3

Ts = Surface temperature of cylinder (K)

Ta = Ambient temperature (K)

### Introduction of The Equipment

###### Figure 8.1:  Schematic Diagram of Experiment

Simultaneous heat transference by conduction and convection (free or forced) is the base of several industrial heat exchangers, as well as the several devices related to them. This equipment of EDIBON allows studying the efficiency of different exchangers, analyzing the heat transmission coefficients of each of the exchangers exposed to different airflows. A fan placed in the upper part of the tunnel allows

controlling the airflow that goes through the tunnel. An interface connectable to a PC contains the control circuits to measure temperatures, electrical control, electrical supply and speed control of the fan.

The interface provides an output to a PC where the values and graphics of all the variables involved in the practices are shown. The airflow is measured with a flowmeter set on the inferior part of the tunnel. This EDIBON equipment allows making a study of the heat transmission in three different types of exchangers

- Flat exchanger

- Tenon exchanger

- Fins exchanger

### Description of The Equipment

The equipment consists of a stainless-steel tunnel of rectangular section supported by a structure in anodized aluminium that allows to be set on the worktable. In the tunnel, three types of different heat exchangers can be set: flat exchanger, tenon exchanger and fins exchanger. Each exchanger has an electric

resistance for its heating. In the three bases of the exchangers we set a thermocouple that serves to control the temperature that the exchangers reach. The heat exchanger that is used in each moment can be observed by a methacrylate window placed in the opposite side of the tunnel The rising air flux through the tunnel can be generated by a variable speed fan set on the superior part. The airflow generated in the tunnel is measured by a flow meter placed in the inferior part of the tunnel. By some air flux concentrators a correct distribution of the airflow is guaranteed by the area of heat exchange. Two thermocouples measure the air temperature at the inlet and outlet of the area of heat exchange. Temperature measurements, at different distances of the base of the tenon and fins exchangers, are made by other five thermocouples that are introduced by one side of the tunnel. These measurements allow observing the gradients of temperature

in the longitudinal direction of the exchangers and in the direction of the air flux. An interface receives the signal of all the thermocouples and the flow meter, providing a computerized outlet of all the measurements. By the interface we make a control of temperature at the bases of the exchangers. Besides, the fan supply and the electrical resistance are provided by the interface. The electric supply of all the equipment is made by the connection of the interface to the net.

## Objective: 1

To demonstrate the relationship between power input and surface temperature in free convection

### Procedure

- We will start taking the measure of the surface that exchanges heat.

- Next, we place the flat exchanger in the tower

- We insert the thermocouples. A thermocouple will be placed in the exchanger, and the other three in the three orifices placed in the middle of the tunnel.

- The equipment is connected. The power of the resistance is regulated in fixed values.

- When the temperatures are established we will proceed to its measurement, as well as that of the air speed.

- Once all the measures are taken the equipment is unplugged. When some time has gone by, the thermocouples and the exchanger are removed

- Repeat the experiment by turning the fan in “ON” position

### Observations

| No | AR-1 (Ohm) | SC-1 (l/s) | ST-1 (°C) | ST-2 (°C) | ST-3 (°C) | ST-4 (°C) | ST-5 (°C) | ST-6 (°C) | ST-7 (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### Table 11.1: Temperatures in free convection

| No | AR-1 (Ohm) | SC-1 (l/s) | ST-1 (°C) | ST-2 (°C) | ST-3 (°C) | ST-4 (°C) | ST-5 (°C) | ST-6 (°C) | ST-7 (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### Table 11.2: Temperatures in forced convection

### Graph

Draw the Graph of temperatures (y-Axis) and AR-1 (X- Axis) for free as well as in forced convection

### Conclusion

## Objective: 2

Measure dependence of the heat transmission on exchanger geometry

### Procedure

- You will begin placing the blade exchanger in the tower. Next, the thermocouples are introduced to measure the temperature in the base of the exchanger, that of the inlet air and that of outlet.

- Fix the electric power of the resistance and the speed of the fan at fixed values

- Once the measures are taken, repeat the experiment with the spike exchanger.

### Observations:

|  | No | AR-1 (Ohm) | SC-1 (l/s) | ST-1 (°C) | ST-2 (°C) | ST-3 (°C) | ST-4 (°C) | ST-5 (°C) | ST-6 (°C) | ST-7 (°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blade Heat exchanger |  |  |  |  |  |  |  |  |  |  |
| spike exchanger |  |  |  |  |  |  |  |  |  |  |

##### Table 11.3: Temperatures for different geometry of exchangers

### Graph

Draw the Graph of temperatures (y-Axis) and AR-1 (X- Axis)

### Conclusion

## Specimen Calculation

NIL

## Statistical Analysis

NIL

## Questions

- Write is difference on temperatures in free and forced convection. Explain

- What is free and forced convection

- What is effect on heat transfer by changing the geometry of exchanger

## Comments

# LAB SESSION 12

To calculate logarithmic mean temperature difference (LMTD), global heat transfer coefficient and effectiveness of turbulent flow heat exchanger

## Learning Objective

- Balance the energy in the Turbulent Flow exchanger.

- Calculate the heat exchanger effectiveness by NTU method.

## Apparatus

Turbulent Flow Heat Exchanger (TIFTC) (Serial no TIFT 0003/06)

## Main Parts

| IDENTIFICATION | DESCRIPTION |
| --- | --- |
| ST-1 | Temperature sensor in the water tank. |
| ST-2 | Cold water temperature sensor at the exchanger inlet or outlet |
| ST-3 | Hot water sensor at the exchanger inlet |
| ST-4 | Cold water sensor between the first and second stretch of the exchanger |
| ST-5 | Hot water sensor between the first and second stretch of the exchanger |
| ST-6 | Cold water sensor between the second and third stretch of the exchanger |
| ST-7 | Hot water sensor between the second and third stretch of the exchanger |
| ST-8 | Cold water temperature sensor at the exchanger inlet or outlet |
| ST-9 | Hot water sensor at the exchanger outlet |
| ST-10 | Temperature sensor of the exterior surface of the interior tube at the exchanger Inlet. |
| ST-11 | Temperature sensor of the exterior surface of the interior tube at the exchanger Outlet |
| SC-1 | Hot water flow sensor |
| SC-2 | Cold water flow sensor |
| AVR-1 | Hot water flow regulation valve. |
| AVR-2 | Cold water flow regulation valve |
| AN-1 | Water level switch of the tank |
| AR-1 | Electric resistance |
| AB-1 | Centrifugal pump for hot water circulation |
| AP-1 | Cold water circuit purge |
| AP-2 | Hot water circuit purge |
| AV-2,AV-3, AV-4 and AV-5 | Ball valves of the cold water circuit to fix parallel or crosscurrent flow |
| AV-1,AV-6, AV-7 and AV-8 | Ball valves to drain the pipes |

## Useful Data

Exchange Length: L = 3 × 0.5 = 1.5 m.

### Interior Tube

Internal Diameter: Dint = 8×10- 3 m

External Diameter: Dext = 10×10- 3 m

Thickness = 10- 3 m

Internal heat transference area: Ah = 0.0377 m2

External heat transference area: Ac = 0.0471 m2

### Exterior Tube

Internal Diameter: Dint,c =13×10- 3 m

External Diameter: Dext,c =15×10- 3 m

Thickness = 10- 3 m

### Base Unit

Net weight: 30 kg.

Height: 400 mm

Width: 1000 mm

Depth: 500 mm

### Heat Exchanger

Net weight: 20 kg.

Height: 200 mm

Width: 1000 mm

Depth: 500 mm

### Physical Properties of The Hot And Cold Water

To determine their physical properties, the average temperature of each fluid has to be calculated.

Hot water average temperature: Tmh  = ST3+ST92

Cold Water Average Temperature: Tmc =ST2+ST82

From the table of the appendix A, the physical properties based on the average temperature can be obtained

|  | 𝜌 | Cp=Specific heat(J/kgK) | 𝜇= Dynamic viscosity (kg/ms) |
| --- | --- | --- | --- |
| Hot water at Tmh °C | 𝜌h= | Cph | 𝜇h |
| Cold water at Tmc °C | 𝜌c= | Cph | 𝜇c |

### Mass Flow Rates

The  mass  flows  of  both  fluids,  are  going  to  be  obtained  from  the measurements taken in the flow sensors (SC1 for hot water and SC2 for cold water)

m (kg/s) = 𝜌.SC= ρ kgm3.SC(lmin)60×1000

Mass flow for hot water= mh (kg/s) = 𝜌.SC= ρh kgm3.SC(litremin)60×1000=……..kg/s

                Mass flow for cold water= mc (kg/s) = 𝜌.SC= ρc kgm3.SC(litremin)60×1000=……..kg/s

###### Figure 12.1: Schematic Diagram of Experiment

## Theory

### Heat transference in heat exchangers

A heat exchanger is a device developed by humans for the heat transference between two fluids at different temperatures separated by a solid wall. They have many engineering applications and, as a consequence, there are many models adapted to each application. The simplest one is the one built with two concentric tubes, where fluids can move in the same sense or in the opposite one. In parallel flow, the hot and the cold water go in and out through the same end. In crosscurrent flow, the fluids go in and out through opposite ends and they circulate in opposite senses.

### Overall Heat-Transfer Coefficient

The overall heat-transfer coefficient U is defined by the relation

q=UA∆Toverall                      Equation 12.1       Overall heat transfer coefficient

Although final heat-exchanger designs will be made on the basis of careful calculations of U, it is helpful to have a tabulation of values of the overall heat-transfer coefficient for various situations that may be encountered in practice

### The Log Mean Temperature Difference (LMTD)

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flow

And

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in counter current flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]             Equation 12.2: The Log Mean Temperature Difference

### Effectiveness-Ntu Method

Effectiveness= Actual heat transferMaximum possible heat transfer                    Equation 12.3

### Capacity coefficient

Capacity coefficient will be defined as (CR)

Cr=CminCmax=………. w/kw/k                                               Equation 12.4

Mass flow rate multiplied by specific heat) Ch and Cc for the hot and cold fluids respectively, and denoting the smaller one as Cmin

### Reynolds number

Reynolds number (Re) is a dimensionless quantity that is used to help predict similar flow patterns in different fluid flow situations. The Reynolds number is defined as the ratio of momentum forces to viscous forces and consequently quantifies the relative importance of these two types of forces for given flow conditions. They are also used to characterize different flow regimes within a similar fluid, such as laminar or turbulent flow

- Laminar flow occurs at low Reynolds numbers, where viscous forces are dominant, and is characterized by smooth, constant fluid motion;

- Turbulent flow occurs at high Reynolds numbers and is dominated by inertial forces, which tend to produce chaotic eddies, vortices and other flow instabilities.

Readers can read further detail from book “heat transfer by J.P Holman” in “chapter 10”

## Objective 01

Balance the energy in the Turbulent Flow exchanger.

### Procedure:

- Check that the valves are opened and that we have crosscurrent flow configuration.

- Check that the heating tank is full of water, above the level switch

- Switch on the pump and the resistor (Equipment supply)

- Fix the tank temperature at 50 °C (STI)

- Fix the hot water flow in 3 l/min (SCI) and adjust the cold-water flow until reaching stationary operation conditions maintaining constant the temperature fixed in the tank.

- Note down the temperature and flow measures in the experimental sheet

- Repeat steps 5 and 6 for different water temperatures in the deposit 55 °C, 60°C and 65 °C.

- Once the measures are made, Calculate the heat transferred by the hot water, the heat absorbed by the cool water, the heat loses, the logarithmic mean temperature and global heat transference coefficient

### Observations

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| ST1 (°C) |  |  |  |  |
| ST2 (°C) |  |  |  |  |
| ST3 (°C) |  |  |  |  |
| ST4 (°C) |  |  |  |  |
| ST5 (°C) |  |  |  |  |
| ST6 (°C) |  |  |  |  |
| ST7 (°C) |  |  |  |  |
| ST8 (°C) |  |  |  |  |
| ST9 (°C) |  |  |  |  |
| ST10 (°C) |  |  |  |  |
| ST11 (°C) |  |  |  |  |
| SC1 (l/min) | 3 | 3 | 3 | 3 |
| SC2 (l/min) |  |  |  |  |

##### Table 12.1: Temperature at different points of turbulent flow heat exchanger

### Calculated Data

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| qh (w) |  |  |  |  |
| qc (w) |  |  |  |  |
| ql (w) |  |  |  |  |
| ∆Tlm (K) |  |  |  |  |
| U (w/m2k) |  |  |  |  |

##### Table 12.2: Heat transfer calculation in turbulent flow heat exchanger

### Specimen Calculations

#### Heat transferred by the hot water

qh=mhCph(Thi-Tho)=………………..watt

#### Heat absorbed by the cold water

qc=mcCpc(Tco-Tci)=………………..watt

#### Heat Losses

ql= qh- qc=……………………watt

#### Logarithmic temperatures mean difference between hot water and cold water

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in counter current flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

#### Global heat transference coefficient= (U)

From the heat transfer rate                          Q=UA∆Tlm

U.A=q/∆Tlm= qh/∆Tlm=……………..w/k

If we take an average heat transfererence area

Am=π.L=Dint+Dext2= π.1.5. 8+102000=0.04241m2

Where

Dint=8.10-3m and Dext=10.10-3m are interior and exterior diameters of the interior tube and L=1.5 m is the exchanger length. Finally, global heat transference coefficient will be

U=…….W/KAm=…..w/k0.04241=…w/m2k

## Objective:02

Calculate the heat exchanger effectiveness by NTU method.

### Procedure

- Check that the valves are opened and that we have crosscurrent flow configuration.

- Check that the heating tank is full of water, above the level switch

- Switch on the pump and the resistor (feeding of the equipment)

- Fix the tank temperature at 65 °C (STI)

- Fix the hot water flow in 2.5 l/min (SCI) and adjust the cold-water flow until reaching stationary operation conditions maintaining constant the temperature fixed in the tank.

- Note down the temperature and flow measures in the experimental sheet

- Place conveniently the valves to invert the cold-water flow sense obtaining a parallel flow

- Make sure that the 65 °C are maintained in the tank and that the same hot and cold-water flows fixed in step 5 are also maintained.

- Once the system is stabilized, note down the measures and flows in the experimental sheet.

- Once the measures are taken, calculate the experimental effectiveness, the theoretical effectiveness with the NTU method and theoretical temperatures at the exchanger outlet.

### Observations

|  | TEST 1 Crosscurrent Flow | TEST 2 Parallel Flow |
| --- | --- | --- |
| ST1 (°C) |  |  |
| ST2 (°C) |  |  |
| ST3 (°C) |  |  |
| ST4 (°C) |  |  |
| ST5 (°C) |  |  |
| ST6 (°C) |  |  |
| ST7 (°C) |  |  |
| ST8 (°C) |  |  |
| ST9 (°C) |  |  |
| ST10 (°C) |  |  |
| ST11 (°C) |  |  |
| SC1 (l/min) |  |  |
| SC2 (l/min) |  |  |

##### Table 12.3: Temperature for parallel and cross flow

### Calculations

From these measures, calculate the following thermodynamic variables

Experimental effectiveness (ϵ)

Heat transferred by the hot water (qh)

Logarithmic temperatures mean difference between hot and cold water (∆Tlm)

Parameters: U.A, NTU and CR

Effectiveness obtained by NTU method (ϵNTU)

Hot and cold water temperatures at the exchanger outlet obtained from the experimental effectiveness (Th,˳y,Tc,o)

|  | TEST 1 Crosscurrent Flow | TEST 2 Parallel flow |
| --- | --- | --- |
| ϵ |  |  |
| qh (w) |  |  |
| ∆Tlm (K) |  |  |
| U.A (w/k) |  |  |
| NTU |  |  |
| CR |  |  |
| ϵNTU |  |  |

##### Table 12.4: Effectiveness of turbulent flow exchanger

### Specimen Calculations

The effectiveness is the quotient between the heat really exchanged and the maximum heat that could be transferred in an infinite area exchanger in a crosscurrent flow.

If                                                    mhCph˂mcCpc          ϵ=Th,i-Th,oTh,i-Tc,o

mhCph ˃mcCpc             ϵ=Tc,o-Tc,iTh,i-Tc,i

#### Heat transferred by the hot water

qh=mhCph(Thi-Tho)=………………..watt

#### Logarithmic temperatures mean difference between hot water and cold water

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in counter current flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

From the heat transfer rate                           Q=UA∆Tlm

U.A=q/∆Tlm= qh/∆Tlm=……………..w/k

#### Number of transmission units

Calculating NTU

NTU= ϵ=U.A(m.Cp)min

#### Capacity coefficient

Capacity coefficient is determined by

Cr=mCpminmCpmax=………. w/kw/k

Once the NTU and Cr are obtained, we can calculate the effectiveness, but depending if the flow is in crosscurrent or in parallel flow, we will have to use different expressions

ϵNTU=1-e-NTU.(1+CR)1+CR                  For Parallel flow

ϵNTU=1-e-NTU.(1-CR)1-CR×  e-NTU.(1-CR)                 For Crosscurrent flow

Comparing the value of ϵ with ϵ NTU we will have to observe if both values are similar or, if on the other hand, they are very different one from each other

## Graph

Draw the graph between the temperature distribution for parallel and crosscurrent flow. To do this, place temperature values on the vertical (y-) axis for the hot water, cold water and of the interior tube surface in ºC (T); and in the x-axis place the position along the exchanger in meters (x). Take into account that the exchange length is 1.5m and that there are four points to measure the temperature

## Statistical Analysis

- xavg= x1+x2+x3n

- Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Conclusion

## Questions

- What is the difference between the temperature distribution for parallel and crosscurrent flow?

- What is Global heat transference coefficient

- What is Logarithmic temperatures mean difference

## Comments

# LAB SESSION 13

To calculate logarithmic mean temperature difference (LMTD), global heat transfer coefficient and effectiveness of shell and tube heat exchanger

## Learning Objective

- Balance the energy in the Shell and Tube exchanger

- Calculate the heat exchanger effectiveness by NTU method.

## Apparatus

EDIBON TICT's Shell and Tube exchanger (Serial No= TICT ɸɸ 67/11)

## Main Parts

| IDENTIFICATION | DESCRIPTION |
| --- | --- |
| ST-16 | Water Tank Temperature Sensor |
| ST-1 | Hot Water Temperature sensor at the inlet/outlet of the exchanger |
| ST-2 | Hot Water Temperature Sensor at the outlet of the exchanger |
| ST-3 | Cold Water Temperature Sensor at the inlet/outlet of the exchanger |
| ST-4 | Cold Water Temperature Sensor in the first section of the exchanger |
| ST-5 | Cold Water Temperature Sensor in the second section of the exchanger |
| ST-6 | Cold Water Temperature Sensor in the third section of the exchanger |
| ST-7 | Cold Water Temperature Sensor at the inlet/outlet of the exchanger |
| SC-1 | Hot water flow sensor |
| SC-2 | Cold water flow sensor |
| AVR-1 | Hot water flow regulation valve. |
| AVR-2 | Cold water flow regulation valve |
| AN-1 | Water level switch of the tank |
| AR-1 | Electric resistance |
| AB-1 | Hot Water Flow Centrifugal Pump |
| AP-1 | Cold Water Circuit Purge Valve |
| AP-2 | Hot Water Circuit Purge Valve |
| AV-2,AV-3, AV-4 Y AV-5 | Ball valves of the cold water circuit to fix parallel or crosscurrent flow |
| AV-1, AV-6, AV-7 Y AV-8 | Ball valves to drain the pipes |
| AVS-1 | Safety Valve |
| AVS-2 | Safety Valve |

## Useful Data

Exchange Length: L = 0.5 m.

### Interior Tube

Internal Diameter: Dint = 8×10- 3 m

External Diameter: Dext = 10×10- 3 m

Thickness = 10- 3 m

Internal heat transference area: Ah = 0.0126 m2

External heat transference area: Ac = 0.0157 m2

### Exterior Tube

Internal Diameter: Dint,c =0.148 m

External Diameter: Dext,c =0.160 m

Thickness =6 ×10- 3 m

### Base Unit

Net weight: 30 kg.

Height: 400 mm

Width: 1000 mm

Depth: 500 mm

### Heat Exchanger

Net weight: 20 kg.

Height: 300 mm

Width: 1000 mm

Depth: 500 mm

### Physical Properties Of The Hot And Cold Water

To determine their physical properties, the average temperature of each fluid has to be calculated.

Hot water average temperature: Tmh  = Thi+Tho2

Cold Water Average Temperature: Tmc =Tci+Tco2

From the table of the appendix A, the physical properties based on the average temperature can be obtained

|  | 𝜌 | Cp=Specific heat(J/kgK) | 𝜇= Dynamic viscosity (kg/ms) |
| --- | --- | --- | --- |
| Hot water at Tmh °C | 𝜌h= | Cph | 𝜇h |
| Cold water at Tmc °C | 𝜌c= | Cph | 𝜇c |

### Mass Flow Rates

The  mass  flows  of  both  fluids,  are  going  to  be  obtained  from  the measurements taken in the flow sensors (SC1 for hot water and SC2 for cold water)

m (kg/s) = 𝜌.SC= ρ kgm3.SC(lmin)60×1000

Mass flow for hot water= mh (kg/s) = 𝜌.SC= ρh kgm3.SC(litremin)60×1000=……..kg/s

                Mass flow for cold water= mc (kg/s) = 𝜌.SC= ρc kgm3.SC(litremin)60×1000=……..kg/s

###### Figure 13.1: Schematic Diagram of Experiment

| COUNTERCURRENT FLOW |  |
| --- | --- |
| AV-2 Valve | CLOSED |
| AV-3 Valve | OPEN |
| AV-4 Valve | OPEN |
| AV-5 Valve | CLOSED |
| PARRALLEL FLOW |  |
| AV-2 Valve | OPEN |
| AV-3 Valve | CLOSED |
| AV-4 Valve | CLOSED |
| AV-5 Valve | OPEN |

##### Table 13.1: Valve positions of shell and tube heat exchanger

## Theory

### Heat transference in heat exchangers

A heat exchanger is a device developed by humans for the heat transference between two fluids at different temperatures separated by a solid wall. They have many engineering applications and, as a consequence, there are many models adapted to each application. The simplest one is the one built with two concentric tubes, where fluids can move in the same sense or in the opposite one. In parallel flow, the hot and the cold water go in and out through the same end. In crosscurrent flow, the fluids go in and out through opposite ends and they circulate in opposite senses.

###### Figure 13.2: Parallel and counter parallel flow

### Shell and tube heat exchanger

A shell and tube heat exchanger is a class of heat exchanger designs. It is the most common type of heat exchanger in oil refineries and other large chemical processes, and is suited for higher-pressure applications. As its name implies, this type of heat exchanger consists of a shell (a large pressure vessel) with a bundle of tubes inside it. One fluid runs through the tubes, and another fluid flows over the tubes (through the shell) to transfer heat between the two fluids. The set of tubes is called a tube bundle, and may be composed of several types of tubes: plain, longitudinally finned, etc.

###### Figure 13.3: Shell and tube heat exchanger

### Overall Heat-Transfer Coefficient

The overall heat-transfer coefficient U is defined by the relation

q=UA∆Toverall                                          Overall heat transfer coefficient

Although final heat-exchanger designs will be made on the basis of careful calculations of U, it is helpful to have a tabulation of values of the overall heat-transfer coefficient for various situations that may be encountered in practice

### The Log Mean Temperature Difference (LMTD)

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in countercurrent flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

### Effectiveness-Ntu Method

Effectiveness= Actual heat transferMaximum possible heat transfer

### Capacity coefficient

Capacity coefficient will be defined as (CR)

Cr=CminCmax=………. w/kw/k

Mass flow rate multiplied by specific heat) Ch and Cc for the hot and cold fluids respectively, and denoting the smaller one as C min

### Reynolds number

In fluid mechanics, the Reynolds number (Re) is a dimensionless quantity that is used to help predict similar flow patterns in different fluid flow situations. The Reynolds number is defined as the ratio of momentum forces to viscous forces and consequently quantifies the relative importance of these two types of forces for given flow conditions. They are also used to characterize different flow regimes within a similar fluid, such as laminar or turbulent flow

- Laminar flow occurs at low Reynolds numbers, where viscous forces are dominant, and is characterized by smooth, constant fluid motion;

- Turbulent flow occurs at high Reynolds numbers and is dominated by inertial forces, which tend to produce chaotic eddies, vortices and other flow instabilities.

## Objective 1

Balance the energy in the Shell and Tube exchanger

### Procedure:

1. Check that the valves are opened and that we have parallel flow configuration.

2. Check that the heating tank is full of water, above the level switch.

- Switch on the pump and the resistor (equipment supply).

- Set the tank temperature at 45 ºC (ST16).

- .Fix the hot water flow in about 3 l/min (SC1) and adjust cold water flow until reaching stationary operating conditions keeping the temperature set in the tank constant.

- Write down temperature and flow measurements on the experimental sheet.

- Repeat steps 5 and 6 for different water temperatures in the tank: 50ºC, 55ºC and 60 ºC.

- Once the measurements may have been taken, you may calculate the heat transferred by the hot water, the heat absorbed by the cool water, heat losses, the logarithmic mean temperature and the global heat transfer coefficient

### Observations

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| ST16 | 45 | 50 | 55 | 60 |
| ST1 (°C) |  |  |  |  |
| ST2 (°C) |  |  |  |  |
| ST3 (°C) |  |  |  |  |
| ST4 (°C) |  |  |  |  |
| ST5 (°C) |  |  |  |  |
| ST6 (°C) |  |  |  |  |
| ST7 (°C) |  |  |  |  |
| SC1 (l/min) | 3 | 3 | 3 | 3 |
| SC2 (l/min) |  |  |  |  |

##### Table 13.2: Temperatures at shell and tube heat exchanger

### Calculated Data

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| qh (w) |  |  |  |  |
| qc (w) |  |  |  |  |
| ql (w) |  |  |  |  |
| ∆Tlm (K) |  |  |  |  |
| U (w/m2k) |  |  |  |  |

##### Table13.3: Heat transfer coefficient of shell and tube heat exchanger

### Specimen Calculations

#### Heat transferred by hot water (qh)

qh = mh Cph ( Th,i - Th,o )

#### Heat absorbed by the cold water (qc)

qc = mc Cpc ( Tc,o - Tc,i )

Where mh and mc are the mass expenses, and Cph and Cpc are the specific heats of the hot and cold fluids

Note: Theoretically qh should equal qc but due to environmental energy losses and to instrumental and observation measurement errors, they are not always equal

#### Heat losses (ql)

ql= qh- qc

#### Logarithmic average temperatures difference between hot and cold water (∆Tlm)

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flo

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in countercurrent flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

#### Global heat transfer coefficient (U)

q = U A ∆Tlm

Global Heat Transfer Coefficient multiplied by the Transfer Area will be

U. A=Qh∆Tlm

Note: U can be calculated obtaining an average value of the heat transfer area: Am= 0.00192

## Objective 2

Calculate the heat exchanger effectiveness by NTU method.

### Procedure

1. Check that the valves are opened and that we have countercurrent flow configuration.

2. Check that the heating tank is filled with water above the level switch.

3. Switch on the pump and the resistor (equipment supply).

4. Set the tank temperature in 60 ºC (ST16).

5. Fix the hot water flow in 3 l/min approx. (SC1) and adjust cold water flow to reach stationary operating conditions, keeping constant temperatures set for the tank.

6. Write down the temperature and flow measurements on the experimental sheet.

7. Set the valves appropriately in order to invert cold water flow direction to produce a parallel flow configuration.

8. Make sure that 60ºC temperatures are kept in the tank and that the same hot and cold water flows set in step 5 are also maintained.

9. Once the system is stabilized, write down the temperature measurements and flow values on the experimental sheet.

10. Once the measurements have been taken, calculate the experimental effectiveness, the theoretical effectiveness by the NTU method and the theoretical temperatures at the exchanger outlet.

### Observations

|  | TEST 1 | TEST 2 |
| --- | --- | --- |
| ST16 | 45 | 50 |
| ST1 (°C) |  |  |
| ST2 (°C) |  |  |
| ST3 (°C) |  |  |
| ST4 (°C) |  |  |
| ST5 (°C) |  |  |
| ST6 (°C) |  |  |
| ST7 (°C) |  |  |
| SC1 (l/min) | 3 | 3 |
| SC2 (l/min) |  |  |

##### Table 13.4: Temperatures for shell and tube heat exchanger

### Calculated Data

|  | TEST 1 Crosscurrent Flow | TEST 2 Parallel flow |
| --- | --- | --- |
| ϵ |  |  |
| qh (w) |  |  |
| ∆Tlm (K) |  |  |
| U.A (w/k) |  |  |
| NTU |  |  |
| CR |  |  |
| ϵNTU |  |  |

##### Table 13.5: Effectiveness of shell and tube heat exchanger

### Specimen Calculations

#### Experimental effectiveness (ϵ)

The effectiveness is the quotient between the heat really exchanged and the maximum heat that could be transferred in an infinite area exchanger in a crosscurrent flow.

If                                                    mhCph˂mcCpc          ϵ=Th,i-Th,oTh,i-Tc,o

mhCph ˃mcCpc             ϵ=Tc,o-Tc,iTh,i-Tc,i

#### Logarithmic average temperatures difference between hot and cold water (∆Tlm)

Being        ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i in countercurrent flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

#### Global heat transfer coefficient (U)

q = U A ∆Tlm

Global Heat Transfer Coefficient multiplied by the Transfer Area will be

U. A=Qh∆Tlm

#### Number of transmission units

NTU=U.A(m.Cp)min

#### Capacity coefficient

Capacity coefficient is determined by

Cr=mCpminmCpmax=………. w/kw/k

Once the NTU and Cr are obtained, we can calculate the effectiveness, but depending if the flow is in crosscurrent or in parallel flow, we will have to use different expressions

#### Temperatures at the exchanger outlet

ϵNTU=1-e-NTU.(1+CR)1+CR                  For Parallel flow

ϵNTU=1-e-NTU.(1-CR)1-CR×  e-NTU.(1-CR)                 For Crosscurrent flow

## Statistical Analysis

For overall heat transfer coefficient “U”

- xavg= x1+x2+x3n

- Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Conclusion

## Questions

- What is the difference between the temperature distribution for parallel and crosscurrent flow in shell and tube heat exchanger

- What is Global heat transference coefficient

- What is Logarithmic temperatures mean difference

- What is difference of shell and tube heat exchanger from other heat exchangers

## Comments

# LAB SESSION 14

To calculate logarithmic mean temperature difference (LMTD) and global heat transfer coefficient of plate heat exchanger

## Learning Objective

Balance the energy in the Shell and Tube exchanger

## Apparatus

EDIBON’s TIPL Plate Heat Exchanger (Serial No= TIPL 0047/11)

## Main Parts

| IDENTIFICATION | DESCRIPTION |
| --- | --- |
| ST-16 | Water Tank Temperature Sensor |
| ST-1 | Cold Water Temperature Sensor at the inlet/Outlet of the exchanger |
| ST-2 | Hot Water Temperature sensor at the inlet of the exchanger |
| ST-3 | Cold Water Temperature sensor at the inlet/outlet of the exchanger |
| ST-4 | Hot Water Temperature sensor at the outlet of the exchanger |
| SC-1 | Hot water flow sensor |
| SC-2 | Cold water flow sensor |
| AVR-1 | Hot water flow regulation valve. |
| AVR-2 | Cold water flow regulation valve |
| AN-1 | Water level switch of the tank |
| AR-1 | Electric resistance |
| AB-1 | Hot Water Flow Centrifugal Pump |
| AV-2,AV-3, AV-4 Y AV-5 | Ball valves of the cold water circuit for setting parallel or counter current flow |
| AV-1,AV-6, AV- 7 and AV-8 | Ball valves for draining the pipes |

## Useful Data

### BASE UNIT

Net weight: 30 kg.

Height: 400 mm

Width: 1000 mm

Depth: 500 mm

### Heat Exchanger

Net weight: 20 kg.

Height: 300 mm

Width: 1000 mm

Depth: 500 mm

Maximum Flow 12m3/h

Max. Work Pressure 10bar

Max. Work Temperature 100ºC

Minimum. Work Temperature 0ºC

Maximum number of plates 10

Internal Circuit Capacity 0.176 liters

External Circuit Capacity 0.22 liters

Surface 0.32m2

Pressures drop 1.4 m.c.a.

### Physical Properties of The Hot and Cold Water

To determine their physical properties, the average temperature of each fluid has to be calculated.

Hot water average temperature: Tmh  = Thi+Tho2

Cold Water Average Temperature: Tmc =Tci+Tco2

From the table of the appendix A, the physical properties based on the average temperature can be obtained

|  | 𝜌 | Cp=Specific heat(J/kgK) | 𝜇= Dynamic viscosity (kg/ms) |
| --- | --- | --- | --- |
| Hot water at Tmh °C | 𝜌h= | Cph | 𝜇h |
| Cold water at Tmc °C | 𝜌c= | Cph | 𝜇c |

### Mass Flow Rates

The  mass  flows  of  both  fluids,  are  going  to  be  obtained  from  the measurements taken in the flow sensors (SC1 for hot water and SC2 for cold water)

m (kg/s) = 𝜌.SC= ρ kgm3.SC(lmin)60×1000

Mass flow for hot water= mh (kg/s) = 𝜌.SC= ρh kgm3.SC(litremin)60×1000=……..kg/s

                Mass flow for cold water= mc (kg/s) = 𝜌.SC= ρc kgm3.SC(litremin)60×1000=……..kg/s

###### Figure 14.1: Schematic Diagram of Experiment

| COUNTERCURRENT FLOW |  |
| --- | --- |
| AV-2 Valve | CLOSED |
| AV-3 Valve | OPEN |
| AV-4 Valve | OPEN |
| AV-5 Valve | CLOSED |
| PARRALLEL FLOW |  |
| AV-2 Valve | OPEN |
| AV-3 Valve | CLOSED |
| AV-4 Valve | CLOSED |
| AV-5 Valve | OPEN |

##### Table 14.1: Position of valves in plate heat exchanger

## Theory

### Heat transference in heat exchangers

A heat exchanger is a device developed by humans for the heat transference between two fluids at different temperatures separated by a solid wall. They have many engineering applications and, as a consequence, there are many models adapted to each application. The simplest one is the one built with two concentric tubes, where fluids can move in the same sense or in the opposite one. In parallel flow, the hot and the cold water go in and out through the same end. In crosscurrent flow, the fluids go in and out through opposite ends and they circulate in opposite senses.

###### Figure 14.2: Parallel and counter parallel flow

### Plate heat exchanger

Plate heat exchanger is a type of heat exchanger that uses metal plates to transfer heat between two fluids. This has a major advantage over a conventional heat exchanger in that the fluids are exposed to a much larger surface area because the fluids spread out over the plates. This facilitates the transfer of heat, and greatly increases the speed of the temperature change. Plate heat exchangers are now common and very small brazed versions are used in the hot-water sections of millions of combination boilers. The high heat transfer efficiency for such a small physical size has increased the domestic hot water (DHW) flow rate of combination boilers. The small plate heat exchanger has made a great impact in domestic heating and hot-water. Larger commercial versions use gaskets between the plates, whereas smaller versions tend to be brazed.

### Overall Heat-Transfer Coefficient

The overall heat-transfer coefficient U is defined by the relation

q=UA∆Toverall                                           Overall heat transfer coefficient

Although final heat-exchanger designs will be made on the basis of careful calculations of U, it is helpful to have a tabulation of values of the overall heat-transfer coefficient for various situations that may be encountered in practice

### The Log Mean Temperature Difference (LMTD)

Being    ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o        in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i             in countercurrent flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]                   The Log Mean Temperature Difference

## Objective

Balance the energy in the Shell and Tube exchanger

### Procedure

1. Verify that valves are opened and that parallel flow configuration has been set.

2. Verify that the heating tank is filled with water over the level switch.

3. Turn on the pump and the resistance (the equipment power supply).

4. Set the tank temperature in 50ºC (ST16).

5. Set the hot water flow in 3 l/min approx. (SC1) and adjust the cold water flow so stationary operating conditions may be reached keeping the temperature in the tank constant

6. Write down temperature and flow measurements on the experimental sheet.

7. Repeat steps 5 and 6 for different temperatures of the water tank: 55ºC, 60ºC and 65ºC.

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| ST16 | 50 | 55 | 60 | 65 |
| ST1 (°C) |  |  |  |  |
| ST2 (°C) |  |  |  |  |
| ST3 (°C) |  |  |  |  |
| ST4 (°C) |  |  |  |  |
| SC1 (l/min) | 3 | 3 | 3 | 3 |
| SC2 (l/min) |  |  |  |  |

8. Once the measurements have been taken you may calculate the heat transferred by the hot water, the heat absorbed by the cold water, heat losses, the logarithmic average temperatures difference and the heat transfer global coefficient

### Observations

##### Table 14.2: Temperatures of Plate Heat Exchangers

### Calculated Data

|  | TEST 1 | TEST 2 | TEST 3 | TEST 4 |
| --- | --- | --- | --- | --- |
| qh (w) |  |  |  |  |
| qc (w) |  |  |  |  |
| ql (w) |  |  |  |  |
| ∆Tlm (K) |  |  |  |  |
| U (w/m2k) |  |  |  |  |

##### Table14.3: Heat transfer in Plate heat exchanger

### Specimen Calculations

#### Heat transferred by hot water (qh)

qh = mh Cph ( Th,i - Th,o )

#### Heat absorbed by the cold water (qc)

qc = mc Cpc ( Tc,o - Tc,i )

Where mh and mc are the mass expenses, and Cph and Cpc are the specific heats of the hot and cold fluids

Note: Theoretically qh should equal qc but due to environmental energy losses and to instrumental and observation measurement errors, they are not always equal

#### Heat losses (ql)

ql= qh- qc

#### Logarithmic average temperatures difference between hot and cold water (∆Tlm)

Being        ∆T1 = Th,i - Tc,i         and	 ∆T2 = Th,o - Tc,o in parallel flow

∆T1 = Th,i - Tc,o           and	 ∆T2 = Th,o - Tc,i in countercurrent flow.

Tlm=∆T1-∆T2ln⁡[∆T1∆T2]

#### Global heat transfer coefficient (U)

q = U A DTlm

Global Heat Transfer Coefficient multiplied by the Transfer Area will be

U. A=Qh∆Tlm

Note: U can be calculated obtaining an average value of the heat transfer area: Am= 0.192

## Graph

Represent the temperature distribution in counter-current and parallel flow configurations. For that purpose, represent hot and cold water temperature values in ºC (T) on the vertical axis; and the position along the exchanger in meters on the horizontal axis. You should consider the length of the exchanger to be 0.5m and that we have three measure points.

## Statistical Analysis

For overall heat transfer coefficient “U”

- xavg= x1+x2+x3n

- Sx=1n-1((x1-xavg)2+(x2-xavg)2+(x3-xavg)2)

## Conclusion

## Questions

- What is the difference between the temperature distribution for parallel and crosscurrent flow in Plate heat exchanger

- What is Global heat transference coefficient

- What is Logarithmic temperatures mean difference

- What is difference of  Plate heat exchanger from other heat exchangers

## Comments