THERMODYNAMICS II

Engr. ANAM ANWAR

(B.Sc. Mechanical Engineering, University of Engineering & Technology Lahore)

Submitted by:

Name =

Registration No =

Submitted to:

# Preface

In most of the engineering institutions, the laboratory course forms an integral form of the basic course in Thermodynamics II at undergraduate level. The experiments to be performed in a laboratory should ideally be designed in such a way as to reinforce the understanding of the basic principles as well as help the students to visualize the various phenomenon encountered in different applications.

The objective of this manual is to familiarize the students with practical skills, measurement techniques and interpretation of results. It is intended to make this manual self-contained in all respects, so that it can be used as a laboratory manual. In all the experiments, the relevant theory and general guidelines for the procedure to be followed have been given. Tabular sheets for entering the observations have also been provided in each experiment while graph sheets have been included wherever necessary.

It is suggested that the students should complete the computations, is the laboratory itself. However the students are advised to refer to the relevant text before interpreting the results and writing a permanent discussion. The questions provided at the end of each experiment will reinforce the students understanding of the subject and also help them to prepare for viva-voce exams.

# General Instructions to Students

- The purpose of this laboratory is to reinforce and enhance your understanding of the fundamentals of Fluid mechanics and Hydraulic machines. The experiments here are designed to demonstrate the applications of the basic fluid mechanics principles and to provide a more intuitive and physical understanding of the theory. The main objective is to introduce a variety of classical experimental and diagnostic techniques, and the principles behind these techniques. This laboratory exercise also provides practice in making engineering judgments, estimates and assessing the reliability of your measurements, skills which are very important in all engineering disciplines.

- Read the lab manual and any background material needed before you come to the lab. You must be prepared for your experiments before coming to the lab. In many cases you may have to go back to your fluid mechanics textbooks to review the principles dealt with in the experiment.

- Actively participate in class and don’t hesitate to ask questions. Utilize the teaching assistants. You should be well prepared before coming to the laboratory, unannounced questions may be asked at any time during the lab.

- Carelessness in personal conduct or in handling equipment may result in serious injury to the individual or the equipment. Do not run near moving machinery. Always be on the alert for strange sounds. Guard against entangling clothes in moving parts of machinery.

- Students must follow the proper dress code inside the laboratory. To protect clothing from dirt, wear a lab apron. Long hair should be tied back.

- Calculator, graph sheets and drawing accessories are mandatory.

- In performing the experiments, proceed carefully to minimize any water spills, especially on the electric circuits and wire.

- Make your workplace clean before leaving the laboratory. Maintain silence, order and discipline inside the lab.

- Cell phones are not allowed inside the laboratory.

- Any injury no matter how small must be reported to the instructor immediately.

- Wish you a nice experience in this lab

Table of contents

Preface	3

General Instructions to Students	4

List of Experiments	9

List of Figure	10

List of Tables	11

1 LAB SESSION 1	12

1.1 Learning Objective	12

1.2 Apparatus	12

1.3 Main Parts of Reciprocating Compressor	12

1.4 Related Theory	12

1.5 Experimental Procedure	14

1.6 Observation and Calculation	15

1.7 Specimen Calculations	16

1.8 Statistical Analysis	16

1.9 Graph	17

1.10 Conclusion:	17

2 LAB SESSION 2	18

2.1 Learning Objective:	18

2.2 Apparatus	18

2.3 Main Parts of Reciprocating Compressor	18

2.4 Experimental Procedure	18

2.5 Observations and Calculations	18

2.6 Specimen Calculations	19

2.7 Statistical Analysis	20

2.8 Graphs	20

2.9 Conclusion	20

3 LAB SESSION 3	21

3.1 Learning Objective:	21

3.2 Apparatus	21

3.3 Main Parts of Reciprocating Compressor	21

3.4 Related Theory	21

3.5 Experimental Procedure	21

3.6 Observations and Calculations	22

3.7 Specimen Calculations	22

3.8 Statistical Analysis	23

3.9 Graph	23

3.10 Conclusion	24

4 LAB SESSION 4	25

4.1 Learning Objective:	25

4.2 Apparatus	25

4.3 Main Parts of Reciprocating Compressor	25

4.4 Experimental Procedure	25

4.5 Observations and Calculations	25

4.6 Specimen Calculations	26

4.7 Statistical Analysis	27

4.8 Graph	27

4.9 Conclusion	27

5 LAB SESSION 5	28

5.1 Learning Objective:	28

5.2 Apparatus	28

5.3 Main Parts of Reciprocating Compressor	28

5.4 Theory	28

5.5 Experimental Procedure	28

5.6 Observations and Calculations	29

5.7 Specimen Calculations	29

5.8 Statistical Analysis	31

5.9 Graphs	31

5.10 Conclusion	32

6 LAB SESSION 6	33

6.1 Learning Objective:	33

6.2 Apparatus	33

6.3 Main Parts of Reciprocating Compressor	33

6.4 Experimental Procedure	33

6.5 Observations and Calculations	33

6.6 Specimen Calculations	34

6.7 Statistical Analysis	36

6.8 Graphs	36

6.9 Conclusion	36

7 LAB SESSION 7	37

7.1 Learning Objective:	37

7.2 Apparatus	37

7.3 Main Parts of Impulse Turbine	37

7.4 Theory	37

7.5 Experimental Procedure	38

7.6 Observations and Calculations	38

7.7 Specimen Calculations	39

7.8 Statistical Analysis	39

7.9 Graphs	39

7.10 Conclusion	39

8 LAB SESSION 8	40

8.1 Learning Objective:	40

8.2 Apparatus	40

8.3 Main Parts of Impulse Turbine	40

8.4 Experimental Procedure	40

8.5 Observations & Calculations	41

8.6 Specimen Calculations	41

8.7Statistical Analysis	42

8.8 Graphs	42

8.9 Conclusion	42

9 LAB SESSION 9	43

9.1Learning Objective:	43

9.2 Apparatus	43

9.3 Main Parts of Reaction Turbine	43

9.4 Theory	43

9.5 Observations and Calculations	44

9.6 Specimen Calculations	44

9.7 Statistical Analysis	46

9.8 Graphs	46

9.9 Conclusion	46

10 LAB SESSION 10	47

10.1 Learning Objective:	47

10.2 Apparatus	47

10.3Main Parts of Reaction Turbine	47

10.4 Experimental Procedure	47

10.5 Observations & Calculations	47

10.6 Specimen Calculations	49

10.7 Statistical Analysis	49

10.8 Graphs	49

10.9 Conclusion	50

# List of Experiments

| Experiment No. | Description |
| --- | --- |
| Experiment No. 1 | To investigate the variation of air flow rate with compressor ratio (without intercooling)- (ITU LAB) |
| Experiment No. 2 | To investigate the variation of air flow rate with compressor ratio (with intercooling) - (ITU LAB) |
| Experiment No. 3 | To investigate the variation in the volumetric efficiency with compressor ratio (without intercooling) - (ITU LAB) |
| Experiment No. 4 | To investigate the variation in the volumetric efficiency with compressor ratio (with intercooling) - (ITU LAB) |
| Experiment No. 5 | To determine the variation of isothermal efficiency with compressor ratio (without intercooler) - (ITU LAB) |
| Experiment No. 6 | To examine the variation of isothermal efficiency with compressor ratio (with intercooler) - (ITU LAB) |
| Experiment No. 7 | To examine the torque and power characteristics of a single stage impulse turbine- (ETC LAB) |
| Experiment No. 8 | To determine the isentropic efficiency of a single stage impulse turbine- (ETC LAB) |
| Experiment No. 9 | To investigate the torque and power characteristics of a reaction turbine- (ETC LAB) |
| Experiment No. 10 | To determine the isentropic efficiency of a reaction turbine- (ETC LAB) |

# List of Figure

Figure 1.1: Two stage reciprocating compressor ………………………………………..12

Figure 7.1: Impulse Turbine Unit…………………………………………………………38

Figure 9.1: Reaction Turbine Unit…………………………………………………...........45

# List of Tables

Table 1.1: Calculation Compressor Ratio without intercooling	15

Table 1.2: Results	16

Table 2.1: Calculation Compressor Ratio with intercooling	19

Table 2.2: Results	20

Table 3.1: Calculation Compressor Ratio without intercooling	22

Table 3.2: Results	23

Table 4.1: Calculation Compressor Ratio with intercooling	27

Table 4.2: Results	28

Table 5.1: Calculation Compressor Ratio without intercooling	30

Table 5.2: Results	32

Table 6.1: Calculation Compressor Ratio with intercooling	35

Table 6.2: Results	36

Table 7.1: Calculations of shaft power of Impulse Turbine	39

Table 8.1: Calculations of isentropic efficiency of Impulse Turbine	43

Table 7.1: Calculations of shaft power of Reaction Turbine	46

Table 8.1: Calculations of isentropic efficiency of Reaction Turbine	49

# 1 LAB SESSION 1

Investigation of variation of air flow rate with compressor ratio (without intercooling)

## 1.1 Learning Objective

At the end of this study, the student will be able to

- Understand the relationship between air flow rate and compressor ratio.

## 1.2 Apparatus

Two stage reciprocating compressor

## 1.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 1.4 Related Theory

Figure 1.1 Two stage Compressor

Air Compressor

A machine suck low pressure, low temperature air from atmosphere and compresses it to high pressure and high temperature by reciprocating motion of compressor is called a compressor.

Working Principal

Work must be done upon the gas by an external agency. An air compressor takes in air at atmospheric pressure, compresses it at the cost of the work supplied, and delivers the high pressure air to a storage vessel called receiver from which it may be conveyed by the pipe line to a place where the supply of compressed air is required.

Following advantages of compresses air are as follow:

- Compressed air produces smooth translation with more uniform force

- Air does not possess the potential shock hazard of electricity or the potential fire hazard of oils.

- It is a utility that is generated in-house, so owners have more control over it than any other utility.

- It can be used to push a piston, as in a jackhammer.

- Compressed air provides torque and rotation power for pneumatic tools, such as drills, brushes, nut runners, riveting guns, and screwdrivers.

- Common applications are to cool large buildings.

- To operate lift, reams and pump.

- In paint spraying and in air conveying, the dynamic pressure of the air imparts motion.

Types of Compressor

Positive Displacement Compressor

In positive displacement type compressors, compression is achieved by trapping air into an enclosed space and then reducing its volume. Since a fixed amount of air is trapped each time, its pressure rises as its volume is reduced. When the pressure rises to the required level, then it is expelled from the enclosed space and a fresh charge of low-pressure air is drawn in and the cycle continues. Since the flow of air to the compressor is not steady, the positive displacement type compressor is a pulsating flow device. However, since the operating speeds are normally very high the flow appears to be almost steady on macroscopic time scale. Depending upon the construction, positive displacement type compressors can be classified into:

- Reciprocating type

- Rotary type with sliding vanes (rolling piston type or multiple vane type)

- Rotary screw type (single screw or twin-screw type)

Reciprocating Compressors

Reciprocating air compressors compresses air in a cylinder by reciprocating motion of piston and supplies high pressure air with intermittent discharge. Rotary compressors have compressed air at low pressure but with continuous and high discharge. Reciprocating air compressors may be single acting or double acting.

Reciprocating compressors consist of a piston moving back and forth in a cylinder, with suction and discharge valves to achieve suction and compression of air. Its construction and working are somewhat similar to a two-stroke engine, as suction and compression of air are completed in one revolution of the crank. Reciprocating air compressors are positive displacement machines, meaning that they increase the pressure of the air by reducing its volume. This means they are taking in successive volumes of air which is confined within a closed space and elevating this air to a higher pressure. The reciprocating air compressor accomplishes this by a piston within a cylinder as the compressing and displacing element.

## 1.5 Experimental Procedure

Procedure starts by entering fresh air from any source at the inlet of first compressor and conditions like temperature pressure are noted at that condition compressor increases the pressure of air. Output of first stage compressor is then enter to intercooler which has some cooling medium like water to reduce the temperature of compressed air, temperature at the exit of intercooler is nearly equal to the initial temperature. After this output air is used as input of second compressor second compressor increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition.

## 1.6 Observation and Calculation

## Table 1.1: Calculation of compressor ratio without intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height H | Compressor Speed Nc Rpm | Compressor Current |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 1.7 Specimen Calculations

T1 = _____0C, T2 = _____0C, T3 = ______0C, T4 = ____0C

1st stage pressure = P1= _____ KNm-2

2nd stage pressure = P2=_____ KNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273) = _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= ______

## Table 1.2: Results

| Sr.# | qv | 1st stage pressure ratio | 2nd stage pressure ratio | Overall pressure ratio |
| --- | --- | --- | --- | --- |
| 01 |  |  |  |  |
| 02 |  |  |  |  |
| 03 |  |  |  |  |
| 04 |  |  |  |  |
| 05 |  |  |  |  |
| 06 |  |  |  |  |
| 07 |  |  |  |  |

## 1.8 Statistical Analysis

## 1.9 Graph

Graph between Q, 1st stage P/R, 2nd stage P/R and Overall P/R

## 1.10 Conclusion:

# 2 LAB SESSION 2

Investigation of variation of air flow rate with compressor ratio (with intercooling)

## 2.1 Learning Objective:

At the end of this study, the student will be able to

- Understand the relationship between air flow rate and compressor ratio.

## 2.2 Apparatus

Two stage reciprocating compressor

## 2.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 2.4 Experimental Procedure

## Procedure starts by entering fresh air from any source at the inlet of first compressor and conditions like temperature pressure are noted at that condition compressor increase the pressure of air. Output of first stage compressor is then enter to intercooler which has some cooling medium like water to reduce the temperature of compressed air, temperature at the exit of intercooler is nearly equal to the initial temperature. After this output air is used as input of second compressor second compressor increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition.

## 2.5 Observations and Calculations

Table 2.1: Calculation of compressor ratio with intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit of 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | Water inlet T5 oC | Water outlet T6 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height h | Cooling water flow mw g/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 2.6 Specimen Calculations

T1 = ___0C, T2 = __0C, T3 = ___0C, T4 = ___0C, T5= ___0C, T6= ___0C

1st stage pressure = P1= _____ KNm-2

2nd stage pressure = P2=_____ KNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273) = _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= _______

Table 2.1: Results

| Sr.# | qv | 1st stage pressure ratio | 2nd stage pressure ratio | Overall pressure ratio |
| --- | --- | --- | --- | --- |
| 01 |  |  |  |  |
| 02 |  |  |  |  |
| 03 |  |  |  |  |
| 04 |  |  |  |  |
| 05 |  |  |  |  |
| 06 |  |  |  |  |
| 07 |  |  |  |  |

## 2.7 Statistical Analysis

## 2.8 Graphs

Graph between Q, 1st stage P/R, 2nd stage P/R and Overall P/R

## 2.9 Conclusion

# 3 LAB SESSION 3

To investigate the variation in the volumetric efficiency with compressor ratio (without intercooling)

## 3.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of volumetric efficiency with compressor ratio.

## 3.2 Apparatus

Two stage reciprocating compressor

## 3.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 3.4 Related Theory

Volumetric Efficiency

Volumetric efficiency is the ratio of the actual volume of gas drawn into the cylinder to the piston displacement. The volumetric efficiency directly affects the capacity because the capacity depends directly on the cylinder’s piston displacement.

## 3.5 Experimental Procedure

Procedure starts by entering fresh air from any source at the inlet of first compressor and conditions like temperature pressure are noted at that condition compressor increase the pressure of air, after this output of this air is used as input of second compressor second compressor increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition.

## 3.6 Observations and Calculations

Compressor swept volume per revolution = 0.3544×10-3 m3

Table 3.1: Calculation of compressor ratio without intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit of 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height H | Compressor speed Nc Rpm | Compressor Current I Amp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 3.7 Specimen Calculations

T1 = ___0C,T 2 = __0C, T3 = ___0C, T4 = ___0C,

1st stage pressure = P1= _____ kNm-2

2nd stage pressure = P2=_____ kNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273)= _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= _______

Va= (compressor swept volume per revolution × Nc) / 60 = ______ m3/ second

ηv= qvva× 100 = _____ %

Table 3.2: Results

| Sr.# | qv | Va | ɳv | Overall P/R |  |
| --- | --- | --- | --- | --- | --- |
| 01 |  |  |  |  |  |
| 02 |  |  |  |  |  |
| 03 |  |  |  |  |  |
| 04 |  |  |  |  |  |
| 05 |  |  |  |  |  |
| 06 |  |  |  |  |  |
| 07 |  |  |  |  |  |

## 3.8 Statistical Analysis

## 3.9 Graph

Graph between overall pressure ratio and volumetric efficiency.

3.10 Conclusion

# 4 LAB SESSION 4

To investigate the variation in the volumetric efficiency with compressor ratio (with intercooling)

## 4.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of volumetric efficiency with compressor ratio.

## 4.2 Apparatus

## Two stage reciprocating compressor

## 4.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 4.4 Experimental Procedure

Procedure starts by entering fresh air from any source at the inlet of first compressor and conditions like temperature pressure are noted at that condition compressor increase the pressure of air. Output of first stage compressor is then enter to intercooler which has some cooling medium like water to reduce the temperature of compressed air, temperature at the exit of intercooler is nearly equal to the initial temperature. After this output of the air is used input of second compressor second compressor increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition

## 4.5 Observations and Calculations

Compressor swept volume per revolution = 0.3544×10-3 m3

Table 4.1: Calculation of Compressor Ratio with intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit of 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | Water inlet T5 oC | Water outlet T6 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height H | Cooling water flow mw g/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 4.6 Specimen Calculations

T1 = ___0C, T2 = __0C, T3 = ___0C, T4 = ___0C, T5= ___0C, T6= ___0C

1st stage pressure = P1= _____ KNm-2

2nd stage pressure = P2=_____ KNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273) = _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= _______

Va = (compressor swept volume per revolution × Nc) / 60 = ______ m3/ s

ηv= qvva× 100 = _____ %

## Table 4.2: Results

| Sr.# | qv | Vs | ɳv | Overall P/R |  |
| --- | --- | --- | --- | --- | --- |
| 01 |  |  |  |  |  |
| 02 |  |  |  |  |  |
| 03 |  |  |  |  |  |
| 04 |  |  |  |  |  |
| 05 |  |  |  |  |  |
| 06 |  |  |  |  |  |
| 07 |  |  |  |  |  |

## 4.7 Statistical Analysis

## 4.8 Graph

Graph between overall pressure ratio and volumetric efficiency

## 4.9 Conclusion

# 5 LAB SESSION 5

To investigate the variation of isothermal efficiency with compressor ratio (without intercooler)

## 5.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of isothermal efficiency with compressor ratio.

## 5.2 Apparatus

Two stage compressor

## 5.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 5.4 Theory

Isothermal Efficiency

The isothermal efficiency is defined as the ratio of the work input to the isothermal process, to the work input to the actual process between the same inlet and exit pressures.

ηC = Reversible isothermal compressor work/Actual compressor work.

## 5.5 Experimental Procedure

Procedure starts by entering fresh air from any source at the inlet of first stage of compressor and conditions like temperature pressure are noted at that condition compressor increase the pressure of air, after this output of the air is used as input of second stage of compressor. It increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition.

## 5.6 Observations and Calculations

Table 5.1: Calculation of compressor ratio without intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit of 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height H | Compressor Speed Nc Rpm | Compressor Current I Amp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 5.7 Specimen Calculations

T1 = ___0C, T2 = __0C, T3 = ___0C, T4 = ___0C

1st stage pressure = P1= _____ KNm-2

2nd stage pressure = P2=_____ KNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273) = _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= _______

ρ Air = Pi / RTi = ______ kg/ litre

Mass flow of air = m = qv × ρ Air = ______ kg/s

For adiabatic compression

To= Ti PoPi γ-1γ   		or

For the 1st stage compression    T2= T1 P1Pi γ-1γ   = ______K= _______0C

Adiabatic work required for compression

WAdiabatic      = mair RTi γ γ-1PoPi γ-1γ-1

= mair RTi γ γ-1P1Pi γ-1γ-1 = _______ watts

Heat is observed by the cooling water and the heat lost can be determined from

Qwater= mwater Cpwater (T6 - T5)= _____ watts

For the 2nd stage compression    T4= T3 P2P1 γ-1γ   = ________K =________ 0C

Adiabatic work required for compression

WAdiabatic      = mair RT3 γ γ-1P2P1 γ-1γ-1

= _______ watts

Work for multi stage compression is = w1 + w2 + w3+ …

WTotal = ______ watts

Isothermal work=mair RTilnPoPi

Wiso = mRt1ln( P1/ Pa) for first stage = ______ watts

Wiso = mRt3ln( P2/ P1) for second stage =______ watts

Wiso(total)= = mRt1ln( P1/ Pa) + mRt3ln( P2/ P1) = ______ watts

ɳiso= Isothermal work / Actual work = ______ %

## Table 5.2: Results

| No.of obs. | T2 | T4 | Wiso | Wactual | Isothermal efficiency |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

## 5.8 Statistical Analysis

## 5.9 Graphs

- Isothermal efficiency vs 1st stage P/R

- Isothermal efficiency vs 2nd stage P/R

- Isothermal efficiency vs overall  P/R

## 5.10 Conclusion

# 6 LAB SESSION 6

To investigate the variation of isothermal efficiency with compressor ratio (with intercooler)

## 6.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of isothermal efficiency with compressor ratio.

## 6.2 Apparatus

Two stage compressor

## 6.3 Main Parts of Reciprocating Compressor

- 1st stage compressor

- 2nd stage compressor

- Intercooler

- Pressure gauges

- Water purge valve

- Rota-meter

## 6.4 Experimental Procedure

Procedure starts by entering fresh air from any source at the inlet of first stage of compressor and conditions like temperature pressure are noted at that condition compressor increase the pressure of air, after this output of the air is used as input of second stage of compressor. It increases pressure mire to a higher value than first one. Note temperature and pressure readings at this condition.

## 6.5 Observations and Calculations

Table 6.1: Calculation of compressor ratio with intercooling

| No. of obs | Inlet Temperature T1 oC | Air exit of 1st stage T2 oC | Air inlet of 2nd stage T3 oC | Air exist of 2nd stage T4 oC | Water inlet T5 oC | Water outlet T6 oC | 1st stag pressure P1 kNm-2 | 2nd stage pressure P2 kNm-2 | Manometric height H | Cooling water flow mw g/s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 6.6 Specimen Calculations

T1 = ___0C, T2 = __0C, T3 = ___0C, T4 = ___0C

1st stage pressure = P1= _____ KNm-2

2nd stage pressure = P2=_____ KNm-2

Manometric height=h= ______m

Air flow measurement= qv = 4.71384×10-5h×(T1+273) = _______ m3/s

Where qv = air flow volume, h= orifice plate manometer pressure mmWg

Inlet abs =Pia = Pi + Patm = 0 + 101.325 = 101.325 KNm-2

1st stage abs= P1a = P1 + Patm = ______ + 101.325 = _______ KNm-2

2nd stage abs = P2a =P2 + Patm= ______ + 101.325 = _______ KNm-2

1st stage pressure ratio = P1aPia = ______

2nd stage pressure ratio = P2aP1a = ______

Overall pressure ratio =P2aPia= _______

ρ Air = Pi / RTi = ______ kg/ litre

Mass flow of air = m = qv × ρ Air = ______ kg/s

For adiabatic compression

To= Ti PoPi γ-1γ   		or

For the 1st stage compression    T2= T1 P1Pi γ-1γ   = ______K= _______0C

Adiabatic work required for compression

WAdiabatic      = mair RTi γ γ-1PoPi γ-1γ-1

= mair RTi γ γ-1P1Pi γ-1γ-1 = _______ watts

Heat is observed by the cooling water and the heat lost can be determined from

Qwater= mwater Cpwater (T6 - T5)= _____ watts

For the 2nd stage compression    T4= T3 P2P1 γ-1γ   = ________K =________ 0C

Adiabatic work required for compression

WAdiabatic      = mair RT3 γ γ-1P2P1 γ-1γ-1 = _______ watts

Work for multi stage compression is = w1 + w2 + w3+ …

WTotal = ______ watts

Isothermal work=mair RTilnPoPi

Wiso = mRt1ln( P1/ Pa) for first stage = ______ watts

Wiso = mRt3ln( P2/ P1) for second stage =______ watts

Wiso(total)= = mRt1ln( P1/ Pa) + mRt3ln( P2/ P1) = ______ watts

ɳiso= Isothermal work / Actual work = ______ %

## Table 6.2: Results

| No.of obs. | T2 | T4 | Wiso | Wactual | Isothermal efficiency |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

## 6.7 Statistical Analysis

## 6.8 Graphs

- Isothermal efficiency vs 1st stage P/R

- Isothermal efficiency vs 2nd stage P/R

- Isothermal efficiency vs overall  P/R

## 6.9 Conclusion

# 7 LAB SESSION 7

To investigate the Torque and Power characteristics of a single stage impulse turbine.

## 7.1 Learning Objective:

At the end of this study, student will be able to

- Explain the torque and power relationship of impulse turbine.

## 7.2 Apparatus

Impulse turbine unit, two stage compressor

## 7.3 Main Parts of Impulse Turbine

- Pressure gauge

- Rota-meter

- Temperature sensors

- Brake band for applying friction

- Flow control valve

## 7.4 Theory

Figure 7.1: Impulse Turbine Unit

Impulse turbine

In an impulse turbine, the fluid is sent through a nozzle so that most of its mechanical energy is converted into kinetic energy. The high speed jet then impinges on bucket-shaped vanes that transfer energy to the turbine shaft.

The modern and most efficient type of impulse turbine was invented by Lester A.Pelton in 1878. And the rotating wheel is now called a Pelton wheel in his honor. The buckets of the Pelton wheel are designed so as to split the flow in half and turn the flow nearly 180 degrees.

A portion of the outermost part of each bucket is cut out so that the majority of the jet can pass through the bucket that is not aligned with the jet to reach the most aligned bucket. In this way, the maximum amount of momentum from the jet is utilized.

## 7.5 Experimental Procedure

- Prepare the turbine for experiment performance by attaching compressor with it.

- Adjust the throttle valve until the inlet air pressure is the desired value – say 60kN m-2 gauge (this pressure must then be constant throughout the test).

- Unscrew the brake adjusting screw until the turbine runs close to its maximum speed but not exceeding 40,000 rpm.

- When conditions are stable, note the speed, spring balance reading and air flow rate.

- Rotate the brake adjusting screw until the turbine runs at about 85% of the initial speed and when stable repeat observation.

- Repeat in similar decrements of speed until the turbine finally the stalls.

- The test may now be repeated at other constant turbine inlet pressures.

## 7.6 Observations and Calculations

Inlet pressure = 60 kNm-2

Effective radius= 0.0145 m

Table 7.1: Calculation of shaft power of impulse turbine

| No. | Inlet pressure P1 kN/m2 | Inlet Temperature T1 Co | Exit Temperature T2 Co | Rotational Speed N rpm × 103 | Brake Band Force F N | Air flow rate m g/s | Torque M Nm × 10-3 | Shaft Power Ps Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |  |

## 7.7 Specimen Calculations

Torque=M = brake band  force × radius = ______Nm

Shaft power = torque × angular speed= M× (2∏ N)/ 60= ________ kW

## 7.8 Statistical Analysis

## 7.9 Graphs

- Torque vs speed

- Power vs speed

## 7.10 Conclusion

# 8 LAB SESSION 8

To determine the isentropic efficiency of a single stage impulse turbine.

## 8.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of pressure and isentropic efficiency of impulse turbine.

## 8.2 Apparatus

Impulse turbine unit, two stage compressor

## 8.3 Main Parts of Impulse Turbine

- Pressure gauge

- Rota-meter

- Temperature sensors

- Brake band for applying friction

- Flow control valve

## 8.4 Experimental Procedure

- Prepare the turbine for experiment performance by attaching compressor with it.

- Adjust the throttle valve until the inlet air pressure is the desired value – say 60kN m-2 gauge (this pressure must then be constant throughout the test).

- Unscrew the brake adjusting screw until the turbine runs close to its maximum speed but not exceeding 40,000 rpm.

- When conditions are stable, note the speed, spring balance reading and air flow rate.

- Rotate the brake adjusting screw until the turbine runs at about 85% of the initial speed and when stable repeat observation.

- Repeat in similar decrements of speed until the turbine finally the stalls.

- The test may now be repeated at other constant turbine inlet pressures.

## 8.5 Observations & Calculations

Inlet pressure = 60 kNm-2

Effective radius= 0.0145 m

Table 8.1: Calculation of isentropic efficiency of impulse turbine

| No. | Inlet pressure P kN/m^2 | Inlet Temperature T1 Co | Exit Temperature T2 Co | Rotational Speed N rpm ×10^3 | Brake Band Force F N | Air flow rate m g/s | Torque M Nm ×10^-3 | Shaft Power Ps Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |  |

## 8.6 Specimen Calculations

Torque=M = brake band force × radius = ______Nm

Shaft power =P= torque × angular speed= M× (2∏ N)/ 60

Absolute temperature at inlet T1= ____ oC = ____+273= _____ K

Atmospheric pressure = _____ kNm-2

Absolute pressure at inlet = P1 = patm+P= ______ kNm-2

Absolute pressure at exist =P2= patm= ______ kNm-2

Turbine pressure ratio rp = P1/ P2 = _____

Turbine Exist temperature =T2= T1rp γ-1γ = ______ K

Isentropic Power= m Cp (T1-T2)= _______ watts

Efficiency= (Actual Power/Isentropic Power) ×100= ______ %PoPi γ-1γ

## 8.7Statistical Analysis

## 8.8 Graphs

- Inlet pressure vs Isentropic efficiency

- Inlet pressure vs power

## 8.9 Conclusion

# 9 LAB SESSION 9

To investigate the Torque and Power characteristics of a single stage reaction turbine.

## Learning Objective:

At the end of this study, student will be able to

- Explain the torque and power relationship of reaction turbine.

## 9.2 Apparatus

Reaction turbine unit, two stage compressor

## 9.3 Main Parts of Reaction Turbine

- Pressure gauge

- Rota-meter

- Temperature sensors

- Brake band for applying friction

- Flow control valve

## 9.4 Theory

Figure 9.1: Reaction Turbine Unit

Reaction turbine

In reaction turbines the rotation is mainly achieved by the reaction forces created by the acceleration of the water in the runner. The basic principle is the same as a rotating lawn sprinkler in which water enters the arms of the sprinkler at low velocity and leave through the jets at high velocity. The exact manner in which this acceleration is achieved in turbine depends upon the whether the runner is of the propeller, Kaplin, Francis or Deriaz type.

The great variety possible in the geometry of the runner makes Reaction Turbines suitable for a wide range of applications. In all types it is normal for a fraction of the hydraulic pressure to be converted into velocity as it passes through the inlet structure which consists of a spiral casing and a gate apparatus leading to the runner. The power from the water (Pressure and Velocity) is then converted into mechanical power in the runner. It is usual for the exit pressure from the runner to be below atmospheric. This is achieved by using a draft tube.

## 9.5 Observations and Calculations

Inlet pressure = 60 kNm-2

Effective radius= 0.0145 m

Table 9.1: Calculations of shaft power of reaction turbine

| No. | Inlet pressure P1 kN/m^2 | Inlet Temperature T1 Co | Exit Temperature T2 Co | Rotational Speed N rpm *10^3 | Brake Band Force F N | Air flow rate m g/s | Torque M Nm *10^-3 | Shaft Power Ps Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |  |

## 9.6 Specimen Calculations

Torque=M = brake band  force × radius = ______Nm

Shaft power = torque × angular speed= M× (2∏ N)/ 60= ______

## 9.7 Statistical Analysis

## 9.8 Graphs

- Torque vs speed

- Power vs speed

## 9.9 Conclusion

# 10 LAB SESSION 10

To determine the isentropic efficiency of a single stage reaction turbine

## 10.1 Learning Objective:

At the end of this study, student will be able to

- Explain the relationship of inlet pressure and isentropic efficiency of reaction turbine.

## 10.2 Apparatus

Impulse turbine unit, two stage compressor

## 10.3Main Parts of Reaction Turbine

- Pressure gauge

- Rota-meter

- Temperature sensors

- Brake band for applying friction

- Flow control valve

## 10.4 Experimental Procedure

- Prepare the turbine for experiment performance by attaching compressor with it.

- Adjust the throttle valve until the inlet air pressure is the desired value – say 60 kN m-2 gauge (this pressure must then be constant throughout the test).

- Unscrew the brake adjusting screw until the turbine runs close to its maximum speed but not exceeding 40,000 rpm.

- When conditions are stable, note the speed, spring balance reading and air flow rate.

- Rotate the brake adjusting screw until the turbine runs at about 85% of the initial speed and when stable repeat observation.

- Repeat in similar decrements of speed until the turbine finally the stalls.

- The test may now be repeated at other constant turbine inlet pressures.

## 10.5 Observations & Calculations

Inlet pressure = 60 kNm-2

Effective radius= 0.0145 m

Table 10.1: Calculations of isentropic efficiency of reaction turbine

| No. | Inlet pressure P kN/m^2 | Inlet Temperature T1 Co | Exit Temperature T2 Co | Rotational Speed N rpm *10^3 | Brake Band Force F N | Air flow rate m g/s | Torque M Nm *10^-3 | Shaft Power Ps Watts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |
| 5. |  |  |  |  |  |  |  |  |
| 6. |  |  |  |  |  |  |  |  |
| 7. |  |  |  |  |  |  |  |  |
| 8. |  |  |  |  |  |  |  |  |

## 10.6 Specimen Calculations

Torque=M = brake band force × radius = ______Nm

Shaft power =P= torque × angular speed= M× (2∏ N)/ 60 = ________= _________kW

Absolute temperature at inlet T1= ____ oC = ____+273= _____ K

Atmospheric pressure = _____ kNm-2

Absolute pressure at inlet = P1 = patm+P= ______ kNm-2

Absolute pressure at exist =P2= patm= ______ kNm-2

Turbine pressure ratio rp = P1/ P2 = _____

Turbine Exist temperature =T2= T1rp γ-1γ = ______ K

Isentropic Power= m Cp (T1-T2)= _______ watts

Efficiency= (Actual Power/Isentropic Power) ×100= ______ %PoPi γ-1γ

## 10.7 Statistical Analysis

## 10.8 Graphs

- Inlet pressure vs Isentropic efficiency

- Inlet pressure vs power

## 10.9 Conclusion